MATHEMATICAL MODELLING OF CONTROL SYSTEMS The control theory that we deal with here is the linear control theory, where the systems that are under consideration are Dynamic, Linear, and Time Invariant systems. A dynamic system , if the cause and effect are proportional, it is called linear. In a linear system, the principle of superposition applies. That is, the response to several inputs can be calculated by treating one input at a time and adding the results. Such systems will be governed by linear differential equations. A differential equation is linear if the coefficicents are constants (time invariant) or functions only of the independent variable (time varying). Transfer Function The transfer function of a linear, time-invariant system is defined as the ratio of the Laplace tansform of the output (Response function) to the Laplace transform of the input (Driving function) under the assumption that all initial conditions are zero. Transfer Function=Gs= LoutputLinput zero initial conditions - The transfer function of a system is a mathematical model which is an operational method of expressing the differentail equation that relates the output variable to the input variable. - The transfer function is a property of a system itself, independent of the magnitude and nature of the input or driving function. - TF does not provide any information concerning the physical structure of the system. The transfer functions of many physically different systems can be identical. - If the transfer function of a system is known, the output response can be studied for several inputs for understanding the nature of the system. - If the transfer function of a system is unknown, it may be established experimentally by introducing known inputs and studying the output of the system. Once established, the transfer function gives a full description of the dynamic characteristics of the system. A revisit to Laplace Transform Lf(t)=Fs=0∞ft e-stdt Step function; ft=A, for t≥0; Lf(t)=0∞A e-stdt=Ae-st-s0∞=As. If A=1, it is unit step function u(t); Lu(t)=1s. Ramp function; ft=At, for t≥0; Lf(t)=0∞At e-stdt=A te-st-s0∞-A0∞1 .e-st-sdt=As e-st-s0∞=As2 Ltn=n!sn+1. Harmonic function; ft=sinωt; Lsinωt=0∞eiωt-e-iωt2i e-stdt=-i20∞e-s-iωtdt-0∞e-s+iωtdt =-i2 e-s-iωt-s-iω0∞- e-s+iωt-s+iω0∞=-i21s-iω-1s+iω =-i2s+iω-s-iωs2+ω2=-i22iωs2+ω2=ωs2+ω2. ft=cosωt; Lcosωt=ss2+ω2. Translated function; f(t-τ)u(t-τ) f(t)u(t); Lf(t-α)=0∞ft-τ u(t-τ) e-stdt put t-τ=α ≫dt=dα 0∞ft-τ u(t-τ) e-stdt=0∞fα u(α) e-s(α+τ)dα=e-sτ0∞fα u(α) e-sαdα=e-sτ F(s). Pulse function; ft=A, for 0≤t≤τ =A ut-A u(t-τ) τ Aτ τ A; Lf(t)=0∞A.ute-stdt-0∞A u(t-τ)e-stdt =As-e-sτAs=As1-e-sτ The strength of the pulse is the area of rectangle. Lf(t)=Aτs1-e-sτ. Aτ Impulse; gt=limτ→0Aτ: 0≤t≤τ As τ→0, the height Aτ→∞ Aτ The area= A τ; Lg(t)=limτ→0Aτs1-e-sτ=limτ→0Ase-sτs=A If A=1, it is unit impulse function (dirac delta function δ(t)) Lδ(t)=1. Multiplication by e-at; e-atft; Le-atf(t)=0∞e-atf(t) e-stdt=0∞f(t) e-(s+a)tdt=F(s+a). Exponential function; ft=e-at; Lf(t)=0∞e-atu(t) e-stdt=1s+a. Real differential theorem; Lddtf(t); 0∞f(t) e-stdt= fte-st-s0∞-0∞f'(t)e-st-sdt Fs=f(0)s+1s Lf'(t) ≫≫Lf't=sFs-f(0). Ldndtnf(t); Ldndtnf(t)=snFs-sn-1f0-sn-2f'0-…. Real Integration theorem; Lftdt; 0∞ftdt e-stdt= ftdte-st-s0∞-0∞fte-st-sdt Lftdt= ftdtst=0 +1s Fs=Fss+f-10s. Final value theorem If f(t) and f’(t) are Laplace transformable, if F(s) is the Laplace transform of f(t), and limt→∞f(t) exists, then limt→∞f(t)= lims→0sF(s) Lf't=sFs-f0 ≪≫ 0∞ddtf(t) e-stdt=sFs-f(0) lims→0 0∞ddtf(t) e-stdt=lims→0 sF(s)-f(0) lim⁡s→0e-st→1 ≪≫ f(t)0∞=lims→0 sF(s)-f(0) f∞-f0=lims→0 sF(s)-f(0) limt→∞f(t)= lims→0sF(s). Initial value theorem If f(t) and f’(t) are both Laplace transformable, if F(s) is the Laplace transform of f(t), and lims→∞sF(s) exists, then f(0+)= lims→∞sF(s) Lf't=sFs-f0+ ≪≫ 0∞ddtf(t) e-stdt=sFs-f(0+) lims→∞ 0+∞ddtf(t) e-stdt=lims→∞ sF(s)-f(0+) lim⁡s→∞e-st→0 ≪≫ 0=lims→∞ sF(s)-f(0+) f(0+)=lims→∞ sF(s).