B. Tech Degree VI Semester Examination April 2016

ME 1605 CAD/CAM (2012 Scheme)

Answer Key & Scheme

| Part A Marks |  |  |
| --- | --- | --- |
| a | In traditional design process ideas and concepts were converted first into sketches and finally to component drawings, assembly drawings and part lists after a long process of analysis and evaluation. These were then given as inputs to material procurement, planning and manufacturing. Modern design process consists of geometric modelling, engineering analysis, design review and analysis and automated drawing. Geometric models are used for all the downstream process including, analysis, evaluation, engineering planning, manufacturing etc. However 2D drawings in printed form are needed for record keeping, meeting mandatory requirements and for presentation purposes. Most of the CAD packages have automated drafting facility. 3D models are converted into fully dimensioned detailed drawings as per the company standards using the appropriate functions in the CAD package. This process is called automated drafting. | 5 |
| b | In manufacturing and assembly minimum rational work elements are combined in to work stations based on the precedence requirements and cycle time. Line balancing is the process of assigning work elements to work stations subject to the precedence requirement and cycle time constraint. Ideally, all the work stations should have the same total time which is equal to the cycle time. But when assigning different work elements to workstations this is not possible. Line balancing is used to assign work elements to workstations such that total number of workstations is minimized and hence balance delay (nTc-TwcnTc) is kept to the minimum. | 5 |
| c | Spindle speed selection: In conventional machine spindle speed is selected by changing a combination of levers where as in CNC machine spindle speed can be selected by M03 or M04 followed by S#### (# numeric value). Only limited numbers of speeds are available with conventional machines. Positioning: in conventional machine tool is brought to the cutting position manually whereas in CNC machine tool is positioned automatically in rapid speed using G00 code. Tool change: in conventional machine tool is changed manually whereas in CNC machines turrets or tool magazines along with tool changers are used to change tools. Thread cutting: thread cutting requires changing a combination of levers for different pitches and engagement and disengagement of split nut in conventional lathes. Only limited numbers of pitches are possible with conventional machines. In CNC machines thread cutting is automated using G33 code. Contouring: Contouring is impossible or difficult (with special attachments) in conventional machines. Contouring is easy with CNC machines. | 5 |
| d | G codes are preparatory function codes which decides the mode of operation of the CNC machine like movement of slides (rapid-G00, linear interpolation -G01, circular interpolation-G01), absolute or incremental (G90 and G91), units (G70,G71), specification of feed (mm/min-G94, mm/rev-G95) etc. M codes are Machine function codes like machine stop (M00,M01, M30), spindle rotation (M03,M04), tool change (M06), coolant on/off (M07/M09) etc. | 5 |
| e | A canned cycle is a way of conveniently performing repetitive CNC machine operations. Canned cycles automate certain machining functions such as drilling, boring, tapping, pocketing, etc. Example: G81 performs 1) positioning in XY plane 2) positioning in Z direction 3) spindle on, 4) movement to the required depth with specified feed, 5) spindle stop and 6) tool retraction. | 5 |
| f | Static errors are geometric and dimensional errors when the machine is not running. Static errors include positioning errors, repeatability, form errors like guideway straightness, spindle run-out (circularity error) etc. Dynamic errors are the errors induced while the machine is running. These can be caused by the acceleration and deceleration of the slides, cutting load and vibration of the machine. Machine test chart give a clear indication of the static errors whereas on line techniques are used to determine dynamic errors. | 5 |
| g | In machine learning artificial neural networks (ANNs) are a family of models inspired by biological neural networks (the central nervous systems of animals, in particular the brain) which are used to estimate or approximate functions that can depend on a large number of inputs and are generally unknown. In the context of CNC machine neural network can be used to select appropriate cutting parameters depending on the cutting conditions. Inputs from sensors like torque sensors, strain gauges, thermocouples; accelerometers etc. are used for section of cutting speed, speed, coolant etc. The neural network is first trained with different combinations of inputs and appropriate output. In operation, appropriate output will be generated by the neural network based on an unknown combination of inputs based on the training. | 5 |
| f | Accuracy – designates how close a measured value is to the true quantity of what is being measured. Resolution – is the smallest increment the system can display or measure. A system can have a high resolution with poor repeatability and accuracy. Repeatability (precision) – describes how well a system or device can reproduce an outcome in unchanged conditions. | 5 |
| Part B |  |  |
| II | Geometric modelling is the electronic storage and representation of the shapes, dimensions and other features of components, their relative positions and interrelationships. Techniques of computer graphics are used to create and visually represent the geometrical entities. Geometric models are electronic replicas of physical components and systems stored in electronic format using computer graphic techniques. Types 1) wireframe models (explain with figures) 2) solid models- C-rep and B-rep methods (explain with figures) 3) Surface models (explain with figures) | 5 10 |
| III | Line Balancing involves selecting the appropriate combination/grouping of work tasks to be performed at each workstation so that the work is completed in a feasible sequence (i.e., making standardized products sequentially step-by-step along the line of flow) within an acceptable cycle time and that the work is evenly divided among the workstations/workers (i.e., each workstation has approximately the same amount of work). Assembly Line balancing (ALB) is an attractive means for optimal balancing of the assembly lines/flow lines in Mass production and Continuous / repetitive production systems (Large-scale Serial production system). Transfer Mechanism (Definition); Transfer mechanism is the mechanism employed to transfer work parts/assemblies from one work station to another. Transfer methods used are: 1) continuous, 2) synchronous and 3) asynchronous The mechanisms used are: 1) conveyor belt (figure) 2) walking beam (figure) 3) Geneva mechanism (figure) Level of automation (Definition) There are five levels of Automation in a production plant - Device level, the lowest level it includes the actuators, sensors, and other lower devices. - Machine level, the hardware from device level is assembled into individual machines. - Cell or system level, a group of machines or workstations connected and supported by a material handling system, computer, and other equipments needed for a manufacturing process Flowline adjustments May be related to the 1) process technology 2) systems technology Adjustments in process technology includes tool changes, tool adjustments Adjustments in system technology adjusting the conveyor speed, re-positioning limit switches etc. | 3 4 4 4 |
| IV | a) Open Loop b) closed loop b) Classifications 1) Based on control systems i.e. open loop and closed loop 2) Point to Point (PTP) and Contouring 3) Incremental and absolute 4) based on machine operations like 1) turning centres and 2) machining centres | 7 |
| V | Total Length: 60mm Max diameter: 30 mm A work piece of 32mm diameter and 70mm length (5mm for holding+4mm for tool over travel +1mm machining allowance) is selected v=60m/min N=100×vπd=637 f=0.1mm/rev Tool: T1 %1234 (program number) N1 G54 G90 G95* N2 T0101 M03 S637* N3 G00 X35 Z0 M08* (positioning) N4 G01 X0 F0.1* (facing) N5 G00 X30 Z2* N6 G01 Z-62* (turning outside dia) N7 G00 X32 Z2* N8 X25* (positioning for step turning) N9 G01 Z-30* (step turning) N10 Z32. N11 G00 X100 Z500* N12 M02* | 15 |
| VI | Main constituents of the non-productive time in a machining centre are 1) positioning time, 2) tool changing time, 3) inspection and tool adjustment time 4) loading and unloading time. Positioning time is reduced by increasing the traverse speed of slides. This can be achieved by using ball screws and ball nuts, servo motors for feed drives and linear motion guideways. Tool changing time can be reduced by using Automatic Tool Changers(ATC) Inspection and tool adjustment time can be reduced by using job and tool probes Loading and unloading time can be reduced by using Automatic Pallet Changers (APC) | 15 |
| VII | ATC Lathes: turrets are used for automatic tool changing in lathes. There are different types of turrets like VDI and non-VDI turrets. Machining Centres: Tool magazines along with tool changing grippers are used in machining centres. There are different types of tool magazines like chain type and drum type magazines. Tool Changer Turret Tool Magazine and tool changing gripper APC There are different types of APCs. Twin pallet and multi pallet APCs are used in machining centres. Different pallet changing mechanisms are used in APCs. Swing-in and swing-out rotary pallet changers are usually used for twin pallet changers. For multi pallet systems linear type of changing mechanisms are used. The pallets are moved by either by chain mechanisms or pneumatic mechanisms (figures). Tool nose radius is compensated using G41 and G42. The difference between programmed tool radius and actual tool radius is entered in the tool compensation page against the tool number. | 6 6 3 |
| VIII | a) Industrial applications of robots: 1) Welding Programming method used: generally teaching-lead through Payload: welding gun configuration: generally revolute type motion: CP 2) Spray painting Programing: generally teaching-lead through payload: spray gun configuration: generally revolute type motion : CP 3) Loading and unloading Programming: textual (positions are exactly known) payload: changeable configuration: cylindrical, spherical or rectangular motion: PTP 4) Assembly Programming: textual language programming payload: components, changeable configuration: SCARA, PUMA, Cylindrical motion: PTP b) Teach Pendant Teach pendent is used in programming by powered lead through method. There are toggle switches for selecting axes or joint co-ordinates. The axes in tool co-ordinate system or base co-ordinate system can be selected by toggle switches. Each axes or joints can be moved in incremental or continuous mode by using buttons or knobs. The movement of the end effector can be recorded and replayed. The end effectors can be closed or opened by flipping appropriate switches. | 10 5 |
| IX | a) Vision and force sensors Arrays (spatial sampling) of photosensitive elements (pixel) converting light energy into electrical energy are used as vision sensors CCD (Charge Coupled Device): each pixel surface is made by a semiconductor device, accumulating free charge when hit by photons (photoelectric effect); these “integrated” charges are “read-out” by a sequential process (external circuitry) and transformed into voltage levels CMOS (Complementary Metal Oxide Semiconductor): each pixel is a photodiode, directly providing a voltage or current proportional to the instantaneous light intensity, with the possibility of random access to each pixel The force sensor is included for calculating the forces of several functions like the machine loading & unloading, material handling, and so on that are performed by a robot. This sensor will also be a better one in the assembly process for checking the problems. There are several techniques used in this sensor like Joint Sensing, Robot – Wrist Force Sensing, and Tactile Array Sensing. b) Computer Integrated Manufacture (CIM) is concerned with providing computer assistance, control and high level integrated automation at all levels of the manufacturing industries, including the business data processing system, CAD, CAM and FMS, by linking islands of automation into a distributed processing system. The technology applied in CIM makes intensive use of distributed computer networks and data processing techniques, Artificial Intelligence and Database Management Systems. FMS in this aspect plays the role of a highly efficient and “ready to react to random requests” manufacturing facility (e.g. machining, test, assembly, welding, etc.) in this “total concept”. | 7 |