- PERFORMANCE TEST ON SINGLE CYLINDER 4 STROKE DIESEL ENGINE

Introduction

A machine, which uses heat energy obtained from combustion of fuel and converts it into mechanical energy, is known as a Heat Engine. They are classified as External and Internal Combustion Engine. In an External Combustion Engine, combustion takes place outside the cylinder and the heat generated from the combustion of the fuel is transferred to the working fluid which is then expanded to develop the power. An Internal Combustion Engine is one where combustion of the fuel takes place inside the cylinder and converts heat energy into mechanical energy. IC engines may be classified based on the working cycle, thermodynamic cycle, speed, fuel, cooling, method of ignition, mounting of engine cylinder and application.

Diesel Engine is an internal combustion engine, which uses heavy oil or diesel oil as a fuel and operates on two or four stroke. In a 4-stroke Diesel engine, the working cycle takes place in two revolutions of the crankshaft or 4 strokes of the piston. In this engine, pure air is sucked to the engine and the fuel is injected with the combustion taking place at the end of the compression stroke. The power developed and the performance of the engine depends on the condition of operation. So it is necessary to test an engine for different conditions based on the requirement.

Specification

Engine 		: Four stroke single cylinder

Make 			: Kirloskar

Power			: 5 HP

Engine speed		: 1500

Fuel			: Diesel

Bore			: 80 mm

Stroke length		: 110 mm

Observations and calculations:

| Mechanical efficiency ηmech |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Brake thermal efficiency ηbth |  |  |  |  |  |  |
| Specific fuel consumption SFC (kg/kW hr) |  |  |  |  |  |  |
| Brake power BP (kW) |  |  |  |  |  |  |
| Time for 10 cc of fuel collected t (sec) |  |  |  |  |  |  |
| Manometer reading (cm) | h = h1 – h2 |  |  |  |  |  |
|  | h2 |  |  |  |  |  |
|  | h1 |  |  |  |  |  |
| Load applied (kg) | F = W-S |  |  |  |  |  |
|  | S |  |  |  |  |  |
|  | W |  |  |  |  |  |
| Speed N (rpm) |  |  |  |  |  |  |
| Sl No. |  | 1 | 2 | 3 | 4 | 5 |

Starting 		: Cranking

Working cycle		: Four stroke

Method of cooling	: Water cooled

Method of ignition	: Compression ignition

Description of the apparatus:

The test rig is built for loading mentioned below:

- The equipment consists of KIRLOSKAR Diesel Engine (Crank started) of 5hp (3.7kW) capacity and is Water cooled. The Engine is coupled to a Rope Brake Drum Dynamometer for loading purposes. Coupling is done by an extension shaft in a separate bearing house. The dynamometer is connected to the spring load assembly for varying the load.

- Thermocouples are provided at appropriate positions and are read by a digital temperature indicator with channel selector to select the position.

- Rota meters are used for direct measurement of water flow rate to the engine and calorimeter respectively.

- Engine Speed and the load applied at various conditions is determined by a Digital RPM Indicator and spring balance reading.

- A separate air box with orifice assembly is provided for regularizing and measuring the flow rate of air. The pressure difference at the orifice is measured by means of Manometer.

- A volumetric flask with a fuel distributor is provided for measurement and directing the fuel to the engine respectively.

Equations used:

Brake power

BP=2 π N (W-S)D+d29.8160000             (in kW)

Where,

S is the spring balance reading in kg

W is the dead weights in kg

D is the diameter of the brake drum = 0.33m

Calculations:

Brake power

BP=2 π N (W-S)D+d29.8160000= __________________kW

Mass of fuel consumed

mfc=0.82 ×3600×X1000 t= _______________________kghr

Specific fuel consumption

sfc= mfcBP = ___________________kgkW hr

Actual volume of air sucked into the cylinder

Va= Cd A 2 g H 3600= _____________________m3hr

H= h ρw 1000 ρa = _____________

Swept volume

Vs= π d24 L N2 60= _____________________

Volumetric efficiency

ηv= VAVs100 = ______________%

Brake thermal efficiency

ηbth= BP 3600 ×100mfc ×CV= ________________

CV = 45200kJ/kg

Mechanical efficiency

ηmech= BP IP= __________

d is the diameter of rope = 0.02m

N is the speed of engine in rpm

Mass of fuel consumed

mfc=0.82 ×3600×X1000 t            (kghr)

Where,

X is the burette reading in cc

0.82 is the density of diesel in gram per cc

t is the time taken in seconds

Specific fuel consumption

sfc= mfcBP           (kgkW hr)

Actual volume of air sucked into the cylinder

Va= Cd A 2 g H 3600               (m3hr)

Where,

H is the head in meter of air

H= h ρw 1000 ρa

A is the area of orifice

h is the manometer reading in mm

ρw is the density of water = 1000kg/m3

ρa is the density of air = 1.193kg/m3

Cd is the coefficient of discharge = 0.62

Swept volume

Vs= π d24 L N2 60

Where,

d is the diameter of bore = 80 mm

L is the length of stroke = 110 mm

N is the speed of engine in rpm

Volumetric efficiency

ηv= VAVs100          (%)

Brake thermal efficiency

ηbth= BP 3600 ×100mfc ×CV

Where,

CV is the calorific value of diesel = 45200kJ/kg

BP is the brake power in kW

Mechanical efficiency

ηmech= BP IP

Where,

IP is the indicated power in kW

Experimentation:

Aim:

The experiment is conducted to

  - To study and understand the performance characteristics of the engine.

  - To draw Performance curves and compare with standards.

Graphs to be plotted:

  - SFC v/s BP

  - ηbth v/s BP

  - ηmech v/s BP

  - ηvol v/s BP

Precautions:

- Do not run the engine if supply voltage is less than 180V

- Do not run the engine without the supply of water.

- Supply water free from dust to prevent blockage in rotameters, engine head and calorimeter.

- Note that the range for water supply provided is an approximate standard values, however the user may select the operating range to his convenience not less than 3 & 2 LPM for engine and calorimeter respectively.

- Do not forget to give electrical earth and neutral connections correctly.

- It is recommended to run the engine at 1500rpm otherwise the rotating parts and bearing of engine may run out.

Procedure:

  - Give the necessary electrical connections to the panel.

  - Check the lubricating oil level in the engine.

  - Check the fuel level in the tank.

  - Allow the water to flow to the engine and the calorimeter and adjust the flow rate.

  - Release the load if any on the dynamometer.

  - Open the three-way cock so that fuel flows to the engine.

  - Start the engine by cranking.

  - Allow to attain the steady state.

  - Load the engine by slowly tightening the yoke rod handle of the Rope brake drum.

  - Note the following readings for particular condition,

    - Engine Speed

    - Time taken for ____cc of diesel consumption

    - Rotameter reading.

    - Manometer readings, in cm of water &

    - Temperatures at different locations.

  - Repeat the experiment for different loads and note down the above readings.

  - After the completion release the load and then switch off the engine.

  - Allow the water to flow for few minutes and then turn it off.

Expected curve:

Brake power

Specific fuel consumption

Brake thermal efficiency

Mechanical efficiency

Volumetric efficiency

Result:

- PERFORMANCE TEST ON FOUR CYLINDER 4 STROKE DIESEL ENGINE

Introduction

A machine, which uses heat energy obtained from combustion of fuel and converts it into mechanical energy, is known as a Heat Engine. They are classified as External and Internal Combustion Engine. In an External Combustion Engine, combustion takes place outside the cylinder and the heat generated from the combustion of the fuel is transferred to the working fluid which is then expanded to develop the power. An Internal Combustion Engine is one where combustion of the fuel takes place inside the cylinder and converts heat energy into mechanical energy. IC engines may be classified based on the working cycle, thermodynamic cycle, speed, fuel, cooling, method of ignition, mounting of engine cylinder and application.

Diesel Engine is an internal combustion engine, which uses heavy oil or diesel oil as a fuel and operates on two or four stroke. In a 4-stroke Diesel engine, the working cycle takes place in two revolutions of the crankshaft or 4 strokes of the piston. In this engine, pure air is sucked to the engine and the fuel is injected with the combustion taking place at the end of the compression stroke. The power developed and the performance of the engine depends on the condition of operation. So it is necessary to test an engine for different conditions based on the requirement.

Specification

Engine 		: Four stroke four cylinder water cooled diesel engine

Make 			: Ambassador classic

Power			: 40 BHP @ 4000 rpm

Torque 		: 8.6 kg. m @ 2000 rpm

Fuel			: Diesel

Bore			: 73 mm

Stroke length		: 88.9 mm

Observations and calculations:

| Mechanical efficiency ηmech |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Brake thermal efficiency ηbth |  |  |  |  |  |  |
| Volumetric efficiency ηv |  |  |  |  |  |  |
| Specific fuel consumption SFC (kg/kW hr) |  |  |  |  |  |  |
| Brake power BP (kW) |  |  |  |  |  |  |
| Time for 10 cc of fuel collected t (sec) |  |  |  |  |  |  |
| Manometer reading (cm) | h = h1 – h2 |  |  |  |  |  |
|  | h2 |  |  |  |  |  |
|  | h1 |  |  |  |  |  |
| Speed N (rpm) |  |  |  |  |  |  |
| Sl No. |  | 1 | 2 | 3 | 4 | 5 |

Starting 		: Self start

Compression ratio	: 23:1

Displacement volume	: 1489 cc

Method of ignition	: Compression ignition

Orifice diameter	: 25 mm

DESCRIPTION OF THE APPARATUS:

The test rig is built for loading mentioned below:

- The equipment consists of Ambassador Diesel Engine (self-started) of 40 BHP capacity and is Water cooled. The Engine is coupled to an AC generator for loading purposes. Coupling is done by an extension shaft in a separate bearing house.

- Thermocouples are provided at appropriate positions and are read by a digital temperature indicator with channel selector to select the position.

- Rota meters are used for direct measurement of water flow rate to the engine and calorimeter respectively.

- Engine Speed at various conditions is determined by a Digital RPM.

- A separate air box with orifice assembly is provided for regularizing and measuring the flow rate of air. The pressure difference at the orifice is measured by means of Manometer.

- A volumetric flask with a fuel distributor is provided for measurement and directing the fuel to the engine respectively.

- The engine test rig is directly coupled to an AC generator, 3 phase, 10kW, 50Hz, which inturn is loaded by resistance load bank. The load can be varied in steps of ¼ , ½ , ¾ and full load by operating the rotary switches provided in the load bank panel

Equations used:

Brake power

BP=3 VIcoscos ∅              (in kW)

Where, V is the voltage across two phase

I is the current in each phase

∅ is the power factor

Calculations:

Brake power

BP=3 VIcoscos ∅ = __________________kW

Mass of fuel consumed

mfc=0.82 ×3600×X1000 t= _______________________kghr

Specific fuel consumption

sfc= mfcBP = ___________________kgkW hr

Actual volume of air sucked into the cylinder

Va= Cd A 2 g H 3600= _____________________m3hr

H= h ρHg 1000 ρa = _____________

Swept volume

Vs= π d24 L N2 603= _____________________

Volumetric efficiency

ηv= VAVs100 = ______________%

Brake thermal efficiency

ηbth= BP 3600 ×100mfc ×CV= ________________

CV = 42500kJ/kg

Mechanical efficiency

ηmech= BP IP= __________

Mass of fuel consumed

mfc=0.82 ×3600×X1000 t            (kghr)

Where,

X is the burette reading in cc

0.82 is the density of diesel in gram per cc

t is the time taken in seconds

Specific fuel consumption

sfc= mfcBP           (kgkW hr)

Actual volume of air sucked into the cylinder

Va= Cd A 2 g H 3600               (m3hr)

Where,

H is the head in meter of air

H= h ρHg 1000 ρa

A is the area of orifice

h is the manometer reading in mm

ρw is the density of water = 1000kg/m3

ρa is the density of air = 1.193kg/m3

Cd is the coefficient of discharge = 0.62

Swept volume

Vs= π d24 L N2 604

Where,

d is the diameter of bore = 73 mm

L is the length of stroke = 88.9 mm

N is the speed of engine in rpm

Volumetric efficiency

ηv= VAVs100          (%)

Brake thermal efficiency

ηbth= BP 3600 ×100mfc ×CV

Where,

CV is the calorific value of diesel = 45200kJ/kg

BP is the brake power in kW

Mechanical efficiency

ηmech= BP IP

Where,

IP is the indicated power in kW

Experimentation:

Aim:

The experiment is conducted to

- To study and understand the performance characteristics of the engine.

- To draw Performance curves and compare with standards.

Graphs to be plotted:

- SFC v/s BP

- ηbth v/s BP

- ηmech v/s BP

- ηvol v/s BP

Precautions:

- Do not run the engine if supply voltage is less than 180V

- Do not run the engine without the supply of water.

- Supply water free from dust to prevent blockage in rotameters, engine head and calorimeter.

- Note that the range for water supply provided is an approximate standard values, however the user may select the operating range to his convenience not less than 3 & 2 LPM for engine and calorimeter respectively.

- Do not forget to give electrical earth and neutral connections correctly.

- It is recommended to run the engine at 1500rpm otherwise the rotating parts and bearing of engine may run out.

Procedure:

  - Give the necessary electrical connections to the panel.

  - Check the lubricating oil level in the engine.

  - Check the fuel level in the tank.

  - Allow the water to flow to the engine and the calorimeter and adjust the flow rate.

  - Release the load if any on the AC generator.

  - Open the three-way cock so that fuel flows to the engine.

  - Start the engine.

  - Allow to attain the steady state.

  - Load the engine by operating the rotary switches on the resistance of the load bank, in steps of ¼, ½, ¾, and full load.

  - Note the following readings for particular condition,

    - Engine Speed

    - Time taken for ____cc of diesel consumption

    - Rotameter reading.

    - Manometer readings, in cm of water &

  - Repeat the experiment for different loads and note down the above readings.

  - After the completion release the load and then switch off the engine.

  - Allow the water to flow for few minutes and then turn it off.

Expected curve:

Brake power

Specific fuel consumption

Brake thermal efficiency

Mechanical efficiency

Volumetric efficiency

Result:

- LOAD TEST ON 3 - CYLINDER 4 - STROKE PETROL ENGINE

Introduction

A machine, which uses heat energy obtained from combustion of fuel and converts it into mechanical energy, is known as a Heat Engine. They are classified as External and Internal Combustion Engine. In an External Combustion Engine, combustion takes place outside the cylinder and the heat generated from the combustion of the fuel is transferred to the working fluid which is then expanded to develop the power. An Internal Combustion Engine is one where combustion of the fuel takes place inside the cylinder and converts heat energy into mechanical energy. IC engines may be classified based on the working cycle, thermodynamic cycle, speed, fuel, cooling, method of ignition, mounting of engine cylinder and application.

Description of the apparatus:

a) Hydraulic Dynamometer Loading

- The equipment consists of a Maruti 800 Engine (Self started) of capacity 800cc and power 12HP.

- The Engine is coupled to a Hydraulic Dynamometer for loading purposes. The coupling is done by a universal coupling in a bearing house.

- Thermocouples are provided at appropriate positions and are read by a digital temperature indicator with channel selector.

- Rotameters are used for direct measurement of water flow rate to the engine and calorimeter respectively.

- The Engine Speed and the load applied at various conditions is determined by a Digital RPM Indicator and Load cell arrangement.

- A separate air box with orifice assembly is provided for regularizing and measuring the flow rate of air. The pressure difference is measured by means of a Manometer.

Observations and calculations:

| Mechanical efficiency ηmech |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Brake thermal efficiency ηbth |  |  |  |  |  |  |
| Volumetric efficiency ηv |  |  |  |  |  |  |
| Specific fuel consumption SFC (kg/kW hr) |  |  |  |  |  |  |
| Brake power BP (kW) |  |  |  |  |  |  |
| Time for 10 cc of fuel collected t (sec) |  |  |  |  |  |  |
| Manometer reading (cm) | h = H1 – H2 |  |  |  |  |  |
|  | H2 |  |  |  |  |  |
|  | H1 |  |  |  |  |  |
| Load W (kg) |  |  |  |  |  |  |
| Speed N (rpm) |  |  |  |  |  |  |
| Sl No. |  | 1 | 2 | 3 | 4 | 5 |

- A burette with a fuel distributor is provided for measurement and directing the fuel to the engine respectively.

Engine specification

Engine 		: Maruti 800

BHP			: 12HP

Speed			: 2000rpm

Fuel			: Petrol

No. of cylinders	: 3

Bore			: 68.5mm

Stroke length		: 72mm

Starting		: Self start

Working cycle		: four stroke

Method of cooling	: water cooled

Method of ignition	: spark ignition

Instrumentation

Digital RPM indicator to measure the speed of the engine.

Digital temperature indicator to measure temperatures at various points.

Differential manometer to measure quantity of air sucked into cylinder.

Burette with stop cock to measure the rate of fuel consumed.

Spring balance to measure load applied to the engine.

b) Fuel measurement

The fuel is supplied to the engine from the main fuel tank through a graduated measuring fuel gauge (Burette) with 2-way cock. To measure the fuel consumption of the engine, fill the burette by opening the cock. By starting a stop clock, measure the time taken to consume 10cc of fuel by the engine.

c) Air intake measurement

The suction side of the engine is connected to an Air tank. The atmospheric air is drawn into the engine cylinder through the air tank. The manometer is provided to measure the pressure drop across an orifice provided in the intake pipe of an air tank. This pressure drop is used to calculate the volume of air drawn into the cylinder. (Orifice diameter is 25mm)

Calculations:

Brake power

BP=W . NC= __________________kW

Mass of fuel consumed

mfc=0.72 ×3600×X1000 t= _______________________kghr

Specific fuel consumption

sfc= mfcBP = ___________________kgkW hr

Actual volume of air sucked into the cylinder

Va= Cd A 2 g H 3600= _____________________m3hr

H= h ρw 1000 ρa = _____________mm

h = H1 – H2 = __________mm

Swept volume

Vs= π d24 L N2 603= _____________________

Volumetric efficiency

ηv= VAVs100 = ______________%

Brake thermal efficiency

ηbth= BP 3600 ×100mfc ×CV= ________________

CV = 43500kJ/kg

Mechanical efficiency

ηmech= BP IP= __________

d) Lubrication

The engine is lubricated by mechanical lubrication.

Lubricating oil recommended – SAE – 40 or equivalent.

e) Temperature measurement

A digital temperature indicator with selector switch is provided on the panel to read the temperature in degree centigrade, directly sensed by respective thermocouples located at different places on the test rig.

Note: use the selector knob gently.

Thermocouple details

T1 : Inlet water temperature of engine jacket and calorimeter

T2 : Outlet water temperature of engine jacket

T3 : Outlet water temperature from the calorimeter

T4 : Exhaust gas temperature from calorimeter

f) Water flow measurement

A flowmeter is provided at the inlet jacket to measure the quantity of water allowed into the engine jacket. A rotameter is provided at inlet of calorimeter. Valves are provided to regulate the flow rate of water. Readings can be taken from flow meter in litres and in rotameter in cc/sec.

g) Loading system

The engine is loaded by varying the quantity of water allowed into the dynamometer at constant pressure head. We can vary the load by operating the gat valve provided on the inlet line of the dynamometer. The dynamometer outlet at the bottom can be adjusted depending upon the load conditions.

Experimentation:

Aim:

The experiment is conducted to

- To study and understand the performance characteristics of the engine.

- To draw Performance curves and compare with standards.

Equations used:

Brake power

BP=W . NC        (in kW)

Where,

W is the spring balance reading in kg

N is the speed of engine in rpm

C is a constant, C = 3240

Mass of fuel consumed

mfc=0.72 ×3600×X1000 t            (kghr)

Where,

X is the burette reading in cc

0.72 is the density of petrol in gram per cc

t is the time taken in seconds

Specific fuel consumption

sfc= mfcBP           (kgkW hr)

Actual volume of air sucked into the cylinder

Va= Cd A 2 g H 3600               (m3hr)

Where,

H is the head in meter of air

H= h ρw 1000 ρa

A is the area of orifice

h is the manometer reading in mm

h = H1 – H2

H1 and H2 are the manometer readings in mm

ρw is the density of water = 1000kg/m3

ρa is the density of air = 1.193kg/m3

Cd is the coefficient of discharge = 0.62

Swept volume

Vs= π d24 L N2 603

Where,

d is the diameter of bore = 68.5mm

L is the length of stroke = 72mm

N is the speed of engine in rpm

Volumetric efficiency

ηv= VAVs100          (%)

Brake thermal efficiency

ηbth= BP 3600 ×100mfc ×CV

Where,

CV is the calorific value of petrol = 43500kJ/kg

BP is the brake power in kW

Mechanical efficiency

ηmech= BP IP

Where,

IP is the indicated power in kW

Graphs to be plotted:

- SFC v/s BP

- ηbth v/s BP

- ηmech v/s BP

- ηvol v/s BP

Procedure:

- Check the lubricating oil level.

- Check the fuel level.

- Check and Release the load on the dynamometer if loaded.

- Check the necessary electrical connections and switch on the panel.

- Provide the Battery Connections.

- Allow cooling water to engine and calorimeter.

- Open the three-way cock so that fuel flows to the engine.

- Start the engine using the starter key.

- Set the speed of the engine. (Do not exceed 2000rpm)

- #Now slowly load the engine using the loading wheel of the dynamometer.

- Set the engine speed to before rating.

- Note the following readings.

  - Engine Speed.

  - Time taken for 10cc of petrol consumption

- Rotameter readings.

- Manometer readings, in ‘m’ of water &

- Temperatures.

- #Repeat the experiment for other loadings.

- After the completion release the load (while doing so release the accelerator) and then switch of the engine and the panel.

- *Allow the water to flow for few minutes and then turn it off.

*Note:  Allow water only to the calorimeter and not to the engine.

- Remove earthing connection of the battery and the starter key.

#Note: Calculate the maximum permissible load on the engine using the equation for rated BHP (given in the specification).

Precautions:

- Do not run the engine if supply voltage is less than 180V

- Do not run the engine without the supply of water.

- Supply water free from dust to prevent blockage in rotameters, engine head and calorimeter.

- Note that the range for water supply provided is an approximate standard values, however the user may select the operating range to his convenience not less than 3 & 2 LPM for engine and calorimeter respectively.

- Always set the accelerator knob to the minimum condition and start the engine.

- Do not forget to give electrical earth and neutral connections correctly.

- It is recommended to run the engine below 3000rpm otherwise the rotating parts and bearing of dynamometer may run out.

Expected curve:

Brake power

Specific fuel consumption

Brake thermal efficiency

Mechanical efficiency

Volumetric efficiency

Result:

- Two Stroke Petrol Engine Test Rig

Introduction

Petrol engines are widely used for various applications. Two stroke petrol engines are preferred especially in automobiles, because of their higher power to weight ratio and smooth operation. The DYNAMIC test rig consists of a two stroke petrol engine coupled to rope brake dynamometer. A loading arrangement is provided to load the engine. Various measurements are provided so that performance of the engine at various loads and speeds can be estimated.

Specifications

- Engine			: Two stroke, single cylinder, horizontal air cooled engine

- Power			: 4.5 HP at 6400rpm

- Gear ratios		: 1st gear ratio 14.4

: 2nd gear ratio 9.9

: 3rd gear ratio 6.8

: 4th gear ratio 5.06

- Dynamometer		:A rope brake pulley of diameter 270 mm with rope attached to the spring balance

- Orifice diameter	: 24 mm

- Calibrated burette for fuel intake measurement

- Orifice meter with water manometer for air intake measurement

- Exhaust gas calorimeter to measure heat carried away by exhaust gases

- Multi-channel digital temperature indicator

- Spring balance to measure the load

Equations used:

Brake power

BP= 2 π N  T60000+0.95+blower power          (kW)

Where,

Transmission efficiency is assumed to be 0.95

T = (Load x 9.81) x effective brake radius   (N-m)

Effective brake radius = 0.141m

Mass of fuel consumed

mfc=0.70 ×3600×X1000 t            (kghr)

Where,

X is the burette reading in cc

Observations:

| Sl no | Brake drum speed N (rpm) | Load (kg) | Time for 10 ml fuel t, (sec) | Manometer difference hw (cm) | Time for collecting 1 litre of water from calorimeter tw (sec) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |

| Sl. no | Temperature readings (0C) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Water inlet to calorimeter,T1 | Water outlet from calorimeter, T2 | Exhaust gas entering the calorimeter, T3 | Exhaust gas leaving the calorimeter, T4 |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |

Calculations:

Brake power

BP= 2 π N  T60000+0.95+blower power = ____________kW

Transmission efficiency is assumed to be 0.95

T = (Load x 9.81) x effective brake radius = _______N-m

Effective brake radius = 0.141m

Mass of fuel consumed

mfc=0.70 ×3600×101000 t =___________kghr

Specific fuel consumption

sfc= mfcBP =____________kgkW hr

Heat supplied by fuel

Hf  = mfc x CV  = _________kJ/hr

CV is the calorific value of the petrol = 43890 kJ/kg

0.70 is the density of petrol in gram per cc

t is the time taken in seconds

Specific fuel consumption

sfc= mfcBP           (kgkW hr)

Heat supplied by fuel

Hf  = mfc x CV    (kJ/hr)

Where,

CV is the calorific value of the petrol = 43890 kJ/kg

Heat equivalent to BP

HBP = BP x 3600    (kJ/hr)

Actual mass of air sucked into the cylinder

ma= Cd A ρ 2 g H 3600               (kghr)

Where,

H is the head in meter of air

H= h ρw 1000 ρa

A is the area of orifice = 4.52 x 10-4 m2

h is the manometer reading in mm

h= hwρwρ     (mm)

hw  is the difference in manometer reading (in mm of waters)

ρw is the density of water = 1000kg/m3

ρ  is the density of air = 1.193kg/m3

Cd is the coefficient of discharge = 0.62

Brake thermal efficiency

ηbth= BP 3600 ×100mfc ×CV

Air fuel ratio

AFR= mamfc

Heat equivalent to BP

HBP = BP x 3600 = _________kJ/hr

Actual mass of air sucked into the cylinder

ma= Cd A ρ 2 g H 3600 = ___________kghr

H is the head in meter of air

H= h ρw 1000 ρa = ______________m

A is the area of orifice = 4.52 x 10-4 m2

h is the manometer reading in mm

h= hwρwρ=____________mm

ρw is the density of water = 1000kg/m3

ρ  is the density of air = 1.193kg/m3

Cd is the coefficient of discharge = 0.62

Brake thermal efficiency

ηbth= BP 3600 ×100mfc ×CV = _____________%

Air fuel ratio

AFR= mamfc= ________

Heat carried away by exhaust gases from heat balance of calorimeter

megCpegT3-T4=mwCpwT2-T1

Cpw is the specific heat of water = 4.187 kJ/kg K

Heat carried away by the exhaust gases, on ambient temperature basis is

Heg = mgCpegT3-T1= ________kW

Heat carried away by exhaust gases from heat balance of calorimeter

Heat given by the exhaust gases in calorimeter = Heat gained by water

megCpegT3-T4=mwCpwT2-T1

Where,

meg is the exhaust gas mass flow rate

Cpeg is the specific heat of the exhaust gases

mw is the calorimeter water flow rate

Cpw is the specific heat of water = 4.187 kJ/kg K

Heat carried away by the exhaust gases, on ambient temperature basis is

Heg = mgCpegT3-T1

Aim

To calculate the performance parameters of a two stroke single cylinder petrol engine

Precaution

- Check the water supply and crank case oil level before starting the engine.

- 2% 2T oil must be added to the petrol.

- Never stop the engine on load.

- For first 30 hrs of the engine operation, do not increase engine speed above 4000rpm.

Graphs to be plotted

Plot the graphs for efficiency, mfc, sfc versus BP at various speeds.

Procedure

- Put sufficient petrol along with it put 2% self-mixing 2T oil in the tank.

- Check oil level in the gear box of the engine. If necessary add up SAE-40 oil. Oil level should always be up to the oil filling hole.

- Confirm that the engine is in neutral gear and ignition switch is ON.

- Start water supply to calorimeter and brake drum.

- Pull the choke knob and give a sharp kick to start the engine. As the engine starts, release the chock knob, pull the clutch lever and put the engine in 4th speed gear.

- Slowly increases the accelerator and set the engine speed to 2500 rpm (say).

- Put load on the engine. (say 5 kg)

- Fill the burette with burette filling cock. Note down time for 10 ml fuel consumption, manometer difference, spring balance difference, calorimeter water flow rate and brake drum speed.

- Note down temperatures from T1 to T4.

- Increase the load by twisting the loading screw. Keep the speed constant with the help of accelerator.

- Repeat the procedure for different loads.

Now keep accelerator position constant and go on increasing the load.

- Note down the speed and fuel consumption at each load.

Calculation table

| Sl No. | Brake power BP (kW) | Specific fuel consumption SFC (kg/kW hr) | Brake thermal efficiency ηbth | Heat carried away by the exhaust gas Heg (kW) |
| --- | --- | --- | --- | --- |

Expected curves:

Specific fuel consumption

Brake power

Brake thermal efficiency

Brake power

Mechanical efficiency

Brake power

Volumetric efficiency

Brake power

Result

- Air Blower Test Rig

Introduction

The blower is single stage centrifugal type. The air is sucked from atmosphere from suction side. The slightly compressed air passes through the spiral case, before it comes out through the outlet.

Blowers are used to discharge higher volumes of air at low pressures. These are used in blast furnaces, cupolas, mines, air conditioning plants, etc. The three types of impellers namely: Backward curved (red), Radial (yellow), Forward curved (black) can be fixed internally at a time.

The blower consists of an impeller fabricated to the shapes. The vanes of the impeller is pressed out of the sheet metal and riveted to the two shrouds thus forming into a unique unit. The other impellers are also fabricated in the same fashion and are easily mountable on the shaft. The casing is made of cast iron. The whole contour is volute shape. The smooth contour helps in reducing eddy current losses along the path. The casing is designed in such a way that it can be separated to facilitate mounting or interchanging of impellers.

The shaft is coupled to an induction motor with the help of a belt and having the facility to change three different speeds with the help of a stepped cone pulley. The input power of the motor can be measured by an energy meter provided on the panel.

The flow of air at inlet and outlet is measured by the manometers. The whole probe can be actuated by the screw mechanism. This render to draw profile of the velocity along the diameter of the pipe.

The pitot tube is used to find out the dynamic and static pressure at suction and delivery side. Venturi is used to find out the quantity of air flow rate, with help of water manometer.

Specification

Type 			: Centrifugal type

Motor 			: 3 HP Induction motor

Speed (max)		: 2800 rpm

Type of impellers	: Backward curved (red)

: Radial (yellow)

: Forward curved (black)

Observations:

| Sl no. | Position of valve | Pitot tube 1 (cm) |  |  | Venturimeter readings (cm) |  |  | Time for 5 revolution (sec) | Speed (rpm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | h1 | h2 | hsta | h1 | h2 | hf |  |  |
| 1 | ¼ open |  |  |  |  |  |  |  |  |
| 2 | ½ open |  |  |  |  |  |  |  |  |
| 3 | ¾ open |  |  |  |  |  |  |  |  |
| 4 | Full open |  |  |  |  |  |  |  |  |

Calculations:

Input power to motor

IPm= n 3600t E= ____________kW

E is the energy meter constant = 200 rev/ kW hr

Input power to blower

IPb= IPm ηt = _____________kW

Static head developed due to air (Pitot tube 1)

Hsta= hsta ρw1000 ρa = _________________m of air

ρw is the density of water = 1000 kg/m3

ρa is the density of air = 1.193 kg/m3

Volume flow rate

Q= Cd K 2 g Hf =_____________m3/sec

Cd is the coefficient of discharge of venturimeter = 0.92

g is the acceleration due to gravity = 9.81 m/sec2

K= A1A2A12-A22= ____________m2

Equations used:

Input power to the motor

IPm= n 3600t E                  (kW)

Where,

n is the number of revolutions

t is the time taken in seconds for the five revolutions of the energy meter

E is the energy meter constant = 200 rev/ kW hr

Input power to the blower

Input power to blower

IPb= IPm ηt (kW)

Where,

ηt  is the transmission efficiency = 90%

Static head developed due to air (Pitot tube)

Hsta= hsta ρw1000 ρa               (m of air)

Where,

hsta is the manometer reading from pitot tube at the entry in mm

ρw is the density of water = 1000 kg/m3

ρa is the density of air = 1.193 kg/m3

Volume flow rate

Q= Cd K 2 g Hf          (m3/sec)

Where,

Cd is the coefficient of discharge of venturimeter = 0.92

g is the acceleration due to gravity = 9.81 m/sec2

K= A1A2A12-A22

Area at the entry of venturimeter

A1=πd124         (m2)

Area at the entry of venturimeter

A1=πd124= ____________m2

d1 is the diameter at the entry of venturimeter = 0.15 m

Area at the throat of venturimeter

A2=πd224= ____________m2

d2 is the diameter at the throat of venturimeter = 0.1 m

Pressure head

Hf= hf ρw1000 ρa= ________m of air

Blower output

P= ρa g Q Htotal1000 =_________kW

Htotal  = Hf + Hsta  = _______ m of air

Blower efficiency

ηblower= PIPb= ______%

Overall efficiency

ηoverall= PIPm= ______%

Calculation table

| Sl No. | Blower input power IP (kW) | Blower output power P (kW) | Volume flow rate Q (m3/sec) | Blower efficiency ηblower | Overall efficiency ηoverall |
| --- | --- | --- | --- | --- | --- |

d1 is the diameter at the entry of venturimeter = 0.15 m

Area at the throat of venturimeter

A2=πd224         (m2)

d2 is the diameter at the throat of venturimeter = 0.1 m

Pressure head

Hf= hf ρw1000 ρa               (m of air)

hf is the velocity head in mm read from manometer-3 (venturimeter)

Blower output

P= ρa g Q Htotal1000          (kW)

Where,

Htotal  = Hf + Hsta

Blower efficiency

ηblower= PIPb

Overall efficiency

ηoverall= PIPm

Aim

To conduct a performance test on an air blower to determine the overall efficiency.

Plot the graph for different volume discharge v/s overall efficiency.

Procedure

- Close the gate valve of the blower provided.

- Start the motor using starter.

- Now open the gate valve for the desired flow.

- Note down the venture, pitot tubes and pressure drop readings.

- Measure the speed of the motor using tacho meter.

- Measure the input power consumed by the motor using energy meter corresponding to the time.

- At various stages/positions of gate valve, note down the corresponding readings.

- Repeat the above procedure for three different speed and also for three different impellers.

Expected curves:

Discharge

Valve position

Overall efficiency

Valve position

Speed

Valve position

Result:

- SINGLE STAGE AIR COMPRESSOR TEST RIG

Introduction

A COMPRESSOR is a device, which sucks in air at atmospheric pressure & increases its pressure by compressing it. If the air is compressed in a single cylinder it is called as a Single Stage Compressor. If the air is compressed in two or more cylinders it is called as a Multi Stage Compressor.

In a Two Stage Compressor the air is sucked from atmosphere & compressed in the first cylinder called the low-pressure cylinder. The compressed air then passes through an inter cooler where its temperature is reduced. The air is then passed into the second cylinder where it is further compressed. The air further goes to the air reservoir where it is stored.

Specification

Make 			: Bakgiam

Type 			: Reciprocating type

Stage			: Single stage

Cylinder		: Single cylinder

Bore			: 100mm

Stroke length		: 80mm

Motor type		: Squirrel cage induction motor

Motor rating		: 5 HP

Motor speed		: 1420 rpm

Compressor speed	: 925 rpm

Electrical supply	: 415 V/380 V, 3 phase, 50Hz

Type of starter		: DOL

Belt size		: B 68

Type of lubrication	: Splash

Type of cooling	: Air cooled

Type of fan		: Forced draught

Diameter of orifice	: 20mm

Equations used:

1. Air head causing flow, ha

ha= hwρwaterρa   ( m of air)

| Sl. No. | Compressor Speed, Nc (rpm) | Delivery pressure (bar) absolute | Inlet pressure of compressor P1 (bar) absolute | Outlet pressure of compressor P2 (bar) Absolute | Motor speed, Nm (rpm) | T1 (K) | T2 (K) | Time for ‘n’ pulses/ revolutions of energy meter, T (sec) | Manometer meter reading (m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | h1 | h2 |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |

Observations:

Calculations:

1. Air head causing flow, ha

ha= h1-h2ρwaterρa =  _____________ m of air

2. Actual vol. of air compressed, Qa

Qa= cd a 2 g ha =  _____________m3/s

3. Theoretical volume of air compressed, Qth

Qth= π4D2 L Nc60= ________________m3/s

- Input Power to the motor, IP

IP=n × 3600 K T= ____________kW

- Isothermal Work done, WD

WD= ρaQaRaTalnln R =_____________W

- Compression ratio

R= P2P1= ______

Where,

hw = (h1 – h2) is water column reading in m of water

ρwater is density of the water = 1000 kg/m³

ρair is the density of the air = 1.293 kg/m³

2. Actual vol. of air compressed, Qa

Qa= cd a 2 g ha          (m3/s)

Where,

ha is air head causing the flow in m of air.

Cd is the co-efficient of discharge of orifice = 0.62

a is the area of orifice = (πd²) / 4

Where, d = diameter of orifice = 0.02 m

3. Theoretical volume of air compressed, Qth

Qth= π4D2 L Nc60      (m3/s)

Where,

D is the diameter of the cylinder = 0.10 m.

L is Stroke Length = 0.080 m

Nc is speed of the compressor in rpm

- Input Power to the motor, IP

IP=n × 3600 K T         (kW)

Where, n    = No. of revolutions of energy meter (Say 5)

K   = Energy meter constant, revs/kW-hr

T   = time for 5 rev. of energy meter in seconds

- Volumetric efficiency, ηvol

ηvol = QaQth 100= __________

- Output power of motor, BP

BP= 2 π Nm. s .g . r60 × 1000= ___________kW

- Power of compressor, Pc

IPc=BP ηt= ___________kW

- Isothermal efficiency, ηiso

ηiso= Isothermal work doneIPc 100=____________

- Overall efficiency, ηiso

| Sl no. | Actual volume of air compressed Qa (in m3/s) | Theoretical volume of air compressed Qth (in m³/s) | Isothermal work done (kW) | Power of compressor Pc (kW) | Output power of motor BP (kW) | Isothermal efficiency ηiso | Volumetric efficiency ηvol | Overall efficiency ηoverall |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |

ηoverall= Isothermal work doneIP  100=____________

Calculation table:

- Isothermal Work done, WD

WD= ρaQa RaTalnln R        W

Where,

ρa = Density of the air = 1.293 kg/m³

Qa = Actual volume of air compressed.

Ra = characteristic gas constant = 0.287 kJ/kg K

Ta = Temperature T1 in kelvin

R = Compression ratio

R= P2P1

Where,

P1 = Suction pressure in bar

P2 = Delivery pressure in bar

NOTE: To convert delivery pressure from kg/cm² to kPa    multiply by 98.1

- Volumetric efficiency, ηvol

ηvol = QaQth 100

- Isothermal efficiency, ηiso

ηiso= Isothermal work doneIPc 100

- Overall efficiency, ηiso

ηiso= Isothermal work doneIP 100

- Output power of motor, BP

BP= 2 π Nm s.g . r60 × 1000

Where,

Nm = speed of motor

s = load cell reading in N

r = radius of the arc = 190 mm

g = acceleration due to gravity

- Power of compressor, Pc

Pc=IP ηm

Where,

ηm = transmission efficiency = 80%

Experimentation:

Aim:

The experiment is conducted at various pressures to

    - Determine the Volumetric efficiency.

    - Determine the Isothermal efficiency.

Procedure:

- Check the lubricating oil level in the compressor.

- Start the compressor by switching on the motor.

- The slow increase of the pressure inside the air reservoir in observed.

- Maintain the required pressure by slowly operating the discharge valve (open/close). (Note there may be slight variations in the pressure readings since it is a dynamic process and the reservoir will be filled continuously till the cut-off.)

- Now note down the following readings in the respective units,

Speed of the compressor.

Manometer readings.

Inlet air pressure

Discharge pressure.

Temperatures.

Energy meter reading.

- Repeat the experiment for different delivery pressures.

- Once the set of readings are taken switch of the compressor.

- The air stored in the tank is discharged. Be careful while doing so, because the compressed air passing through the small area also acts as a air jet which may damage you or your surroundings.

- Repeat the above two steps after every experiment.

Graphs to be plotted:

      - Delivery Pressure vs. ηvol

      - Delivery Pressure vs. ηiso

Precautions:

1.  Do not run the blower if supply voltage is less than 380V

2.  Check the direction of the motor, if the motor runs in opposite direction change the phase line of the motor to run in appropriate direction.

3.  Do not forget to give electrical earth and neutral connections correctly.

Expected curves:

Volumetric efficiency

Delivery pressure

Isothermal efficiency

Delivery pressure

Result: