ENGINEERING MECHANICS Mechanics deals with bodies or systems that are subjected to the action of forces. In general, Mechanics can be divided in to three branches as below: Mechanics. Mechanics of Rigid Bodies; Mechanics of deformable bodies; Mechanics of fluids. Statics; Dynamics. Kinematics; Kinetics. Rigid Body All bodies will undergo some amount of deformation when forces are applied, however small it may be. In many situations our interest will be only on the state of the system (equilibrium or motion) rather than the deformation. In such cases, we can neglect the deformations occuring to the body due to the application of the forces. Then, the bodies are assumed to be rigid. Principles of statics - Parallelogram law If two forces P and Q are acting at point A at an angle θ are represented in magnitude and direction by the force vectors AB and AC, their resultant R represented by the force vector AD in magnitude and direction is given by the diagonal of the parallelogram ABDC.; E A B C D P θ R Q α θ. From the ∆AED, AD2=AE2+ED2= (AB+BE)2+ED2 From the right angled ∆BED, sinθ=EDBD ≫≫ED=BDsinθ=Qsinθ ∎ cosθ=BEBD ≫≫BE=BDcosθ=Qcosθ R2=(P+Qcosθ)2+(Qsinθ)2=P2+2PQcosθ+Q2 cos2θ+Q2 sin2θ=P2+2PQcosθ+Q2 R=P2+Q2+2PQcosθ tanα=EDAE=QsinθP+Qcosθ. case i θ=0 A Q P R=P2+Q2+2PQcos0 R=P+Q tanα=QsinθP+Qcosθ=0≫≫ α=0; case i θ=90 Q A P R=P2+Q2+2PQcos90 R=P2+Q2 tanα=QsinθP+Qcosθ=QP ≫α=tan-1QP; case i θ=180 P A Q R=P2+Q2+2PQcos180 R=P-Q. - Equilibrium Law Two forces can be in equilibrium (Resultant fore=0) only if they are equal, opposite, and collinear.; Look at the three cases of parallelogram law discussed above. We can see that resultant R can become zero, only for the third case when P=Q. For all other occasions, there will always be a resultant.. - Principle of superposition We can add to or subtract from a system of forces, another force system in equilibrium without any change in the state of the system.. P Principle of transmissibility A force can be transmitted along its line of action without any change in the state of the system; P P P A B P A A B B. Consider a force P acting at point A on a body. Let B be a point on the body along the line of action of the force. Two equal and opposite forces of magnitude P are added at point B. This addition will not change the state of the system (principle of superposition). Now the two forces marked in black are equal, opposite, and collinear. Hence, they are in equilibrium which can be subtracted from the system (principle of superposition). Finally, we are left with only one force of magnitude P at B. That is, the force P at A is transmitted along its line of action.. - Law of action and reaction When a force is exerted on a rigid support, the support will exert an equal reaction in the opposite direction. A support is preventing the body from moving or rotating. A support will exert a reaction force in a direction opposite to the direction of the motion prevented and a rection moment in a direction opposite to the direction of the rotation prevented.. Reference - Engineering Mechanics: Timoshenko and Young, McGraw Hill - Mechanics for Engineers: Beer & Johnston, McGraw Hill - Engineering Mechanics: Merriam & Kraig, Wiley - Engineering Mechanics: Hibbeler, Pearson