Equations of equilibrium of concurrent force system in a plane We have seen that the resultant of a system of concurrent coplanar forces can be found out using the equation, R=ΣFx2+ΣFy2. If the force system is in equilibrium, then the resultant of the force system should be zero. This is possible only if ΣFx=0 and ΣFy=0. Hence, the equations of equilibrium for concurrent forces in a plane are, ΣFx=0 and ΣFy=0 Example 1 Three balls of radius r=10 cm and weight W=100 N each resting on a trough as shown. Determine the support reactions at A, B, C, and D.; 50cm B A D C. Consider the freebody diagramθ of the whole system ΣFx=0 ≫≫ R3-R4=0 ≫≫ R3= R4 ΣFy=0 ≫≫ R1+R2-100-100-100=0 ≫ R1+R2=300 From the geometry, 10+20cosθ+20cosθ+10=50 ≫cosθ=34 ≫ θ=41.4°; W=100 N W W R31 R1 R2 R41 θ θ. Consider the free-body diagram of ball 1 ΣFx=0 ≫≫Rcos41.4°-R'cos41.4°=0≫≫R=R' ΣFy=0 ≫≫Rsinθ+R'sinθ=100≫≫R=1002×sin41.4=75.6 N Equilibrium of ball 2 ΣFx=0≫≫ R3-Rcos41.4°=0 ≫R3=56.7 N ΣFy=0 ≫≫R1-Rsin41.4°-100=0≫≫ R1=150 N R2=150 N ≪≫ R4=56.7 N; 3 2 1 θ θ W W W R3 R1 R2 R41 θ θ R' R' R R. Example 2 Three balls of radius r=10 cm and weight W=100 N each resting on a smooth floor. Determine the support reactions at A and B and the tension if a string keeps the balls from moving.; D A W=100 N W W R1 R2 C B 50cm. Consider the freebody diagramθ of the whole system ΣFy=0 ≫≫ R1+R2-100-100-100=0 ≫ R1+R2=300 From the geometry, 10+20cosθ+20cosθ+10=50 ≫cosθ=34 ≫ θ=41.4°; θ θ. Consider the free-body diagram of ball 1 ΣFx=0 ≫≫Rcos41.4°-R'cos41.4°=0≫≫R=R' ΣFy=0 ≫≫Rsinθ+R'sinθ=100≫≫R=1002×sin41.4=75.6 N Equilibrium of ball 2 ΣFx=0≫≫ T-Rcos41.4°=0 ≫T=56.7 N ΣFy=0 ≫≫R1-Rsin41.4°-100=0≫≫ R1=150 N R2=150 N; W W W T R1 R2 T 3 2 1 θ θ θ θ R' R' R R. Example 3 Determine the weight of the cylinder A which will keep the system in equilibrium. Neglect the friction in the pulley at B.. Free body diagram of the cylinder C ΣFy=0 ≫≫T-W=0 ≫T=392 N Since, the friction in the pulley B is neglected, the tension in the string CBE will be the same; W=392N T. Free-body diagram of E ΣFx=0 ≫≫Tcos30-T'=0 ≫T'=339.5 N ΣFy=0 ≫≫Tsin30-W'=0 ≫W'=196 N; T=392 N 30° T' W'. Exercise 1. Two smooth pipes, each having a mass of 300 kg, are supported by the forked tines of a tractor as shown. Determine the reactions at the supports A, B, and C.; C B A. Exercise 2. Determine the tension in the cables AB and BC if the cylinder weighs 600 N.. Exercise 3. Determine the tension in the cables AB, BC and AD.. Exercise 4 If each cable can withstand a maximum load of 500 N, what is the maximum possible weight of the suspended cylinder?. Reference - Engineering Mechanics: Timoshenko and Young, McGraw Hill - Mechanics for Engineers: Beer & Johnston, McGraw Hill - Engineering Mechanics: Merriam & Kraig, Wiley - Engineering Mechanics: Hibbeler, Pearson