Friction Friction is a force that resists the movement of two contacting surfaces that slide relative to one another. This force always acts tangent to the surface at the points of contact and is directed so as to oppose the impending or existing relative motion between the surfaces. F N. Consider a block of weight W resting on a rough surface. Let a horizontal force P acts on the block. If the value of P is increased slowly from a low value, the block will not move initially. That is, the applied force P is resisted by an equal force in the opposite direction. This resistive force is developed at the surface of contact which is known as the frictional force F . The force P and friction F produces a moment in the clockwise direction. To maintain equilibrium, there should be an equal and opposite moment which will be developed by W and N. Hence, the line of action of the normal force N shifts to right as shown. The force N can move only up to the edge of the block as we increase the force P. If the value of P increases further, the body tips over. From the diagram, tanϕs=FsN. ϕs is known as angle of friction. As the value of P increases, the value of F also increases and the body remains in equilibrium until it reaches a particular value, after which the body starts moving. Once the body starts moving, the frictional force decreases a bit. When the body is on the verge of motion, the value of friction is known as limiting static friction.. Coulomb’s laws of friction - Frictional force is independent of the area of contact provided the normal force is neither very small nor very high. When two surfaces are in contact, the surface irregularities will lock against each other which is resisting the relative motion. Though it appears that the entire surface is in contact, the actual contact area is only very small. The two surfaces can slide relative to each other only after breaking the locking surface irregularities. That is the reason why the frictional force decreases once the motion begins.. - The frictional force is proportional to the normal force existing between the surfaces. F∝N ≫≫ F=μN The constant μ is known as the coefficient of friction. μ=tanϕs. - For low velocity of sliding, the frictional force is independent of velocity.. Example 1 A body resting on a rough horizontal plane requires a pull of 18 N inclined at 30° to the plane to just to move it. It was found that that a push of 22 N inclined at 30° to the plane just moved the body. Determine the weight of the body and the coefficient of friction between the body and surface.. Case 1 ΣFx=0 ≫≫18cos30-μR=0 ≫≫μR=15.6 ΣFy=0 ≫≫R+18sin30-W=0≫R=W-9 μW-9=15.6 …………………(1). Case 2 ΣFx=0 ≫≫22cos30-μR'=0 ≫≫μR'=19.1 ΣFy=0 ≫≫R'-22sin30-W=0≫R'=W+11 μW+11=19.1 …………….(2). (1)(2) ⋙ μW-9μW+11=15.619.1 ⋙19.1W-9=15.6 W+11 ⋙ W=15.6×11+19.1×919.1-15.6=98.2 N 2⋙ μ=19.198.2+11=0.173 Example 2 Find the value of angle that the surface PQ makes with the horizontal, if block B is about to slide down. The weight of the block A is 100 N and that of B is 300 N. The block A is tied to the wall with a string parallel to PQ. Take μ=0.25 for all contact surfaces. Also find the tension in the string.; Q P θ A B. Consider free body diagram of block A and resolve the forces along the plane and perpendicular to it. ΣFx=0 ≫≫100sinθ+μR-T=0 ≫≫T=100sinθ+μR ΣFy=0 ≫≫R-100 cos θ=0≫R=100 cos θ T=100sinθ+0.25×100 cos θ=100sinθ+25 cos θ Consider free body diagram of block B and resolve the forces along the plane and perpendicular to it. ΣFx=0 ≫≫300sinθ-μR-μR'=0 ≫≫μR'=300sinθ-0.25×100 cos θ ΣFy=0 ≫≫R'-R-300 cos θ=0≫R'=300cosθ+100 cos θ=400 cos θ 0.25×400 cos θ=300sinθ-0.25×100 cos θ 125cosθ=300sinθ ≫≫ tanθ=0.416 ≫≫θ=22.6° T=100sinθ+0.25×100 cos θ=61.5N; θ y x R μR 300 N R' μR' 100 N R μR θ T. 45° Example 3 Two blocks A and B which are identical and weighing 100 N each are supported by a rod at 45° as shown. If the blocks are in limiting equilibrium, find the coefficient of friction assuming it to be same for all surfaces.; B A. Equilibrium of block A ΣFx=0 ≫≫R-Fcos45=0≫≫R=Fcos45 ΣFy=0 ≫≫μR+Fsin45-100=0≫≫F(μ+1)=1002 Equilibrium of block A ΣFx=0 ≫≫Fcos45-μR'=0≫≫μR'=Fcos45 ΣFy=0 ≫≫R'-Fsin45-100=0≫≫ R'=100+F2 μ100+F2=Fcos45≫100μ=F2-Fμ2≫F1-μ=μ1002; FR FR R'R μR'R μRR R 100 N 100 N 45° 45°. From the two equations F(μ+1)F1-μ=1002μ1002 ≫≫ μ2+μ=1-μ ≫≫ μ2+2μ-1=0 ≫≫ μ=0.414 Example 4 A uniform ladder of length 4m and weight 1800 N rests on a horizontal floor and leans against a vertical wall at 45°. The coefficient of friction between the wall and the ladder is 0.3 and that between the ladder the floor is 0.5. A man whose weight is 900 N climbs up the ladder till it starts to slip. Determine to what height he can climb before it starts slipping?. Let the man climbs a distance x before the ladder slips. ΣFx=0≫≫R-0.5R'=0≫≫R=0.5R' ΣFy=0 ≫≫0.3R+R'-1800-900=0≫≫0.3×0.5R'+R'=2700 R'=27000.15+1=2348 N ≪≫ R=1174 N ΣMA=0 ≫ 1800×2cos45+900×x cos 45-R×4sin45-0.3R×4cos45=0 x=1174×4sin45+0.3×1174×4cos45-3600cos45900cos45=2.78 m; P 15° 0.3RR 45° 2m A 1800 N 900 NR x R'R 0.5R'R R. Example 5 A wedge of angle 15° and negligible weight is used to raise a heavy block weighing 2000 N. Determine the minimum horizontal force P necessary to just raise the block, if the coefficient of friction for all surfaces of contact is 0.25.; 2000 N. Equilibrium of the block ΣFx=0≫≫R'cos75+0.25R'cos15-R=0≫≫R=0.5R' ΣFy=0 ≫R'sin75-0.25R'sin15-0.25R-2000=0≫R'=2576.6N Equilibrium of the wedge ΣFx=0≫≫P-R'cos75-0.25R'cos15-0.25R''=0 ΣFy=0 ≫0.25R'sin15-R'sin75+R''=0≫R''=2322.1 N P=1870 N; 0.25R R 15° 75° R’ R’ 0.25 R’ 0.25 R’ P 0.25R’’ 15° R’’. Exercise 1 Two blocks which are connected by a horizontal link AB are supported on rough planes as shown. The coefficient of friction between block A and plane is 0.4. The angle of friction for block B on the inclined plane is 20°. Find the smallest weight for the block A, for which equilibrium can exist, if the weight of block B is 500 N.; A B 30°. Exercise 2 Find the least force P required to cause motion of the system shown to impend. Assume the coefficient of friction between all sliding surfaces as 0.2, weight of block A=150 N and weight of block B=100 N. Neglect the friction in the pulley.; A θ P B 60°. Exercise 3 Referring to the figure, the coefficients of friction are as follows; 0.25 at the floor, 0.3 at the wall, and 0.2 between the blocks. What is the minimum force that will hold the system in equilibrium?; P 60° 1000 N 2000 N. Reference - Engineering Mechanics: Timoshenko and Young, McGraw Hill - Mechanics for Engineers: Beer & Johnston, McGraw Hill - Engineering Mechanics: Merriam & Kraig, Wiley - Engineering Mechanics: Hibbeler, Pearson