Spur Gear Design

Design considerations

Gear tooth design involves primarily the determination of the proper pitch and face width for adequate strength, durability and economy of manufacture. The specification for a gear drive includes the power to be transmitted, the speed of the driver, the velocity ratio and the centre distance.  As the actual sources of power run at high speeds than that required for driven units, the gear drive will be usually a speed reduction one. Occasionally speed increases also required (like in centrifuges)

Points to be considered while designing a gear drive

- The gear should have sufficient strength so that it should not fail at starting torques or under  dynamic loads

- The teeth must have good wear resistance

- The drive should be silent and compact

- Drive should be properly aligned

- Proper lubrication should be provided for the drive

- The material should have high tensile strength, high endurance strength, good wear résistance, low coefficient of friction, and good manufacturability

Allowable stress

Static design stress which is the allowable stress at zero pitch line velocity can be obtained by dividing the ultimate tensile strength by suitable factor of safety. The actual allowable stress due to load transmitted decrease with increase in velocity due to the increase in the effect of shock loads and inertia of the parts at the higher speeds .Due to the uncertainty of the numerous variables involved, obtaining a purely rational expression for variation of allowable stress with pitch line velocity is difficult and an empirical expression known as velocity factor is used. The value of velocity factor is always less than unity.

The tooth of the gear is subjected to fluctuating bending stress as it comes in contact with the

meshing tooth. Since the teeth are subjected to fluctuating stresses, endurance limit stress (σen) is the criterion of design. Therefore, the maximum bending stress is equal to the endurance limit stress of the gear tooth. The endurance limit stress of the gear tooth depends

upon the following factors:

(i) Surface finish of the gear tooth

(ii) Size of the gear tooth

(iii) Reliability used in design

(iv) Stress concentration in the gear tooth

(v) Gears rotating in one direction or both directions

Approximate value of bending stress σ=σen=13σuwhere σu = Ultimate tensile strength of the material. In case of bronze gears, the endurance limit stress is taken as 40% of the ultimate tensile strength.

Number of teeth

There is a limiting value of the minimum number of teeth on the pinion. As the number of teeth decreases, a point is reached when there is interference and the standard tooth profile requires modification. The minimum number of teeth to avoid interference is given by

zmin=2sin2α

where ‘α’ is the pressure angle. Table 12.4 (b) /Pg. 230 gives the values of minimum number of teeth on the pinion for various tooth system.

Hunting tooth

For uniform distribution of tooth wear extra tooth called the hunting tooth is provided on the gear. Suppose that (z1= 20) and (z2 =40), then after every two revolutions of the pinion, the same pair of teeth will engage. If however, z1= 20 and z2 =41, the pinion will rotate41 times before the same pair of teeth will engage again. This will results in more even distribution of wear. For the provision of hunting tooth, it should be permissible to alter the velocity ratio slightly.

In a multi-stage gearbox consisting of two or three stages, the velocity ratio at each stage should not exceed 6: 1. The intermediate speeds, in this case, are arranged in geometric progression. If i’ is the total transmission ratio, i.e., the ratio of angular velocity of the first driving gear to that of the last driven gear, then the speed reduction at each stage (i) is obtained in the following way.

For two stages i=i      for three stagesi=3i

Face width

In the Lewis equation, it is assumed that the tangential force Ft is uniformly distributed over the entire face width. If the face width is too large, there is a possibility of concentration of load at one end of the gear tooth due to a number of factors, like misalignment, elastic deformation of shafts, and warping of gear tooth. On the other hand, gears with a small face width have a poor capacity to resist the shock and absorb vibrations. They also wear at a faster rate. A narrow face width results in a coarse pitch. In practice, the optimum range of the face width is (8 m) <b < (12 m). In the preliminary stages of gear design, the face width is assumed as ten times of module.

Transmitted Torque or Maximum torque (Mt)

In practical applications, the torque developed by the source of power varies during the work cycle. Similarly, the torque required by the driven machine also varies. The two sides are balanced by means of a flywheel. In gear design, the maximum force (due to maximum torque) is the criterion. This is accounted by means of a service factor, Cs.

Cs = Maximum torque/ rated torque

Cs= MtT

Where Mt = Maximum torque or transmitted torque and T = rated torque

Rated torque in N-mmT=9.55×106×Pn  where P = Power in kW and n = speed in rpm                                                                                                              Eqn. 3.3 (a) /Pg. 50

Mt=Cs×T

Service factor, Cs. is given in                             Table 12.8/Pg.235

Beam Strength of Spur Gear Tooth

Assumptions for Lewis Equation

- The tangential component the force Ft on the gear tooth is uniformly distributed across the face width

- The effect of radial component of  Fr which produces direct compressive stresses is neglected

- The maximum stress is assumed to occur when the entire load is at the tip of the tooth.

- The tooth is assumed to be a simple cantilever beam

- The effect of stress concentration and manufacturing errors are neglected

Assuming the tooth cross section to be rectangular shape the moment of inertia I=bt3/12 and c=t/2

The bending stress		σ=Ft×h×t/2bt3 /12

Ft=σbt26h

Multiplying numerator and denominator of the right hand side equation by circular pitch, p,

Ft=σbt2p6hp

Ft=σbt2p6hπm

p=πmEqn. 12.1 (a) /Pg. 203

y=t26πhm		Lewis form factor

Ft=σbyp	Eqn. 12.5 (a) /Pg. 204

Ft=σdCvbYm

Where σd = allowable static stress

Y = form factor = πy

m = module

Cv = Velocity factor

~ 0.5 if velocity can’t be determined

m=FtσdCVbY

Introducing a constant k =b/m, face width b = km. Substituting b in the above equation

m=FtσdCVkmY

m2=FtσdCVkY

Module  m=FtσdCvkY                                    Eqn. 12.5 (b) /Pg. 204

Ft=2Mtd=2Mtzm

Where Mt = Transmitted torque and d = pitch circle diameter = m x z

z = number of teeth

m2=FtσdCVkY=2MtσdCvkYzm

m3=2MtσdCVkYz

m=2MtσdCvkYz13                   Eqn. 12.5(b)/Pg.204

Ft=1000×P×Csv

,                                                                                     Eqn. 12.7 (a) /Pg. 205

Ft= The tangential tooth load in N, σd = Allowable Static stress in N/mm2                                                                            Table 12.7 /Pg. 234

Cv = Velocity factor		                                                           Eqn. 12.6 (a to e) /Pg. 205

k= b/m ~ 10	   b = face width     9.5m≤b≤12.5m                           Eqn. 12.5 (f) /Pg. 205

Y = πy, form factor where y = Lewis form factor              Table (12.5 &12.6) / Pg. 232 &233

Dynamic tooth load

The dynamic loading may be due to the flowing reasons

- The teeth may not be perfectly spaced

- The elements of face are not perfectly parallel to the axis

- The profiles may not be perfectly involute

- Deflection of tooth under load

- Non uniformity of load distribution

- Shaft and mountings deflects under load

Due to these irregularities there will be dynamic load due to shock and impact and its value will be more than the steady transmitted load.

Fd=Ft+F1

=Ft+K3vCb+FtK3v+Cb+FtEqn. 12.12/Pg.207

C = Dynamic factor depending upon machining error

K3 = 20.67

Dynamic strength of the gear

Fs=σdbYm≥Fd	                  Eqn.12.16/Pg.208

Wear load

Gears may wear excessively because of improper or insufficient lubrication, and due to the presence of foreign particles. In addition there will be wear causing pitting of the tooth surfaces due to the compressive fatigue stresses. Properly applied lubrication can eliminate some of these troubles but the teeth must be such that compressive fatigue failure of the material may not take place.

Fw=d1bQK≥FdEqn.12.15 (a)/Pg.208

Where K= load stress factor

Q=2d2d2+d1=2z2z2+z1           The ratio factor                  Eqn.12.15 (c)/Pg.208

Endurance strength of gear

Fen=σenbYm   		                       Eqn.12.13 (a)/Pg.207

Margin of safety of beam strength

Fen = 1.25 Fd for steady loads

= 1.35 Fdfor pulsating loads

= 1.50Fd for shock loads

Design Procedure for Spur  Gears

1. Material selection

- Select suitable material if not given from Table 12.7 &12.16 /Pg. 234 &Pg. 239

- Identifying the weaker part for design (pinion/gear)

- If the material used for both pinion and gear is same, the pinion is weaker

- If the material used for pinion and gear are different then the strength factor (σd y) is used to determine the weaker part where y is the Lewis form factor, Eqn. 12.5 (c to e) /Pg. 204

- Design is based on weaker part

2. To find module

- If diameter is known use m=FtσdCvkY                         Eqn. 12.5 (b) /Pg. 204

Ft, the tangential tooth load                  Ft=1000PCsv

P = Power in kW, Cs = service factor                                                           Table12.8/ Pg. 235

v = pitch line velocity, m/s, σd = Allowable Static stress                            Table 12.7 /Pg. 234 Cv = Velocity factor		                                                           Eqn. 12.6 (a to e) /Pg. 205

k= b/m = face width/ module ~10, b = face width 9.5m≤b≤12.5m     Eqn. 12.5(f)/Pg.205

Y = πy, form factor where y = Lewis form factor                                      Table 12.5 / Pg. 232

OR

b. If the diameter is unknown

Module m=2MtσdCvkYz13                                                              Eqn. 12.5(b)/Pg.204

where Mt = transmitted torque 	= Rated torque x Service factor Cs

c. Select the standard value of module                                 Table (12.2) / Pg. 229

d. Calculate induced stress from the equation  Ft=σinducedbyp

Where, p = circular pitch = πm                                                                   Eqn. 12.1(a)/Pg.203

e. Calculate the allowable stress, σdCv

f. for safe design, induced stress <allowable stress, otherwise increase the module and repeat the calculation until σ induced <   σdCv

g. Face width may be re calculated as bnew = b x induced stress / allowable stress

3.Calculate Dynamic tooth load (Fd)

Fd=Ft+F1

Eqn. 12.12/Pg.207

=Ft+K3vCb+FtK3v+Cb+Ft

C = Dynamic factor depending upon machining error, ‘e’        Table (12.12) / Pg. 236

e is the measured error in action between gears in mm

K3 = 20.67Pg. 207

4.Calculate Dynamic strength of the gear(Fs)

Fs=σdbYm≥Fd			         Eqn.12.16/Pg.208

5.Calculate the wear load(Fw)

Fw=d1bQK≥FdEqn.12.15 (a)/Pg.208

Where K= load stress factor    
K=σes2  sinα1.41E1+1E2

Table.12.16 and 12.17/Pg.239 &240

σes =  the surface endurance limit σes=2.75BHN-70

BHN is the average Brinell hardness number of gear and pinion for the steels

Q=2d2d2+d1=2z2z2+z1      the ratio factor           Eqn.12.15 (c)/Pg.208

Where d2&d1 the pitch circle diameters of gear & pinion and z2&z1 the number of teethes in gear and pinion respectively.

- For safe design Fw > Fd

If Fw< Fd, then equate Fw= Fd= d1bQK, Find the value of K and choose suitable hardness for gear and pinion using                                             (Table.12.16 and 12.17/Pg.239 &240)

6. Calculate the Endurance strength of gear(Fen)

Eqn.12.13 (a)/Pg.207

σen = Endurance limit                                                                                   Table.12.15/Pg.238

For safe design Fen> F d Margin of safety of beam strength

Fen = 1.25 Fd for steady loads

= 1.35 Fd for pulsating loads

= 1.50Fd for shock loads

Recommended series of modules in mm Table 12.2 /Pg.229

Standard tooth proportions of involute spur gears Table 12.3 /Pg.229 or Table 12.4 (a) /Pg.230

Numericals ( Type – I problems : Number of teeth is given )

Q1. A cast steel spur gear pinion having 21 teeth and rotating at 1500 rpm is required to transmit 9 kW to a high grade CI gear to run at 500 rpm with light shocks for 8 to 10 hours per day. The teeth are 14 ½ degree involute. The allowable static stresses for pinion and gear are 135 and 103 MPa respectively. Design the gear from stand point of strength only.

Given Data

No of teeth of pinion, z1 = 21, n1 = 1500 rpm, Power, P = 9 kW, n2 = 500 rpm

α = pressure angle = 14 ½ degree involute, allowable static stresses for pinion

σd1 = 135 MPa = 135 N/mm2, allowable static stresses for gear σd2 = 103 MPa = 103 N/mm2. Design the gear from stand point of strength only

Speed ratio

z1 = 21, z2 = number of teeth on gear = i x z1 = 3 x 21 = 63

- Identifying the weaker part for design (pinion/gear)

Since the material used for pinion and gear are different then the strength factor (σd y) is used to determine the weaker part where y is the Lewis form factor

For 14 ½ degree Lewis form factor

Eqn. 12.5 (c) /Pg. 204

y1 = 0.0914 and y2= 0.113

σd1x y1 = 135 x 0.0914 = 12.34 N/mm2, σd2x y2 = 103 x 0.113 = 11.64 N/mm2. Since the capacity for the gear is less, gear is the weaker and the design is based on gear.

Module

Since diameter are unknown

m=                                                                           Eqn. 12.5(b)/Pg.204

Where Mt = transmitted torque

Rated torque in N-mm

T=9.55×106×Pn2=9.55×106×9500=171900Nmm

Mt = transmitted torque = Rated torque in N-mm x Service factor Cs

Cs for light shocks for 8 to 10 hours per day = 1.25                     Table.12.8/Pg.235

Mt    = 171900x1.25 = 214875 Nmm, σd =σd2 = 103 N/mm2

Assume k =b/m =10, Cv =0.5, Y =π y2=0.113 π, z =z2 =63

m=2MtσdCvkYz1/3=2×214875103×0.5×10×0.113×π×631/3=3.34mm

m = module = 4 mm = d/z                                                           Table.12.2/Pg.229

Pitch circle diameter of gear, d2 = m x z2 =4 x 63 = 252 mm

Transmitted load, Ft = 2Mt/d2

Ft=2Mtd2= 2×214875252=1705.34 N

Velocity,

For ordinary cut gears running with a velocity up to 8 m/s the velocity factor Cv

Eqn. 12.6(a)/Pg.205

Cv=3.053.05+v=3.053.05+6.6=0.316

Allowable stress

Induced stress

where b = face width = k x m = 10 x 4 = 40 mm, y = form factor =y2 = 0.113

p= circular pitch = π x m = 4π mm

σinduced=Ftbyp=1705.3440×0.113×4×π=30.32 Nmm2

Since the induced stress < allowable stress, the design is safe from the point of view of strength.

Velocity ratio =3

m = module = 4 mm

Number of teeth, Pinion = 21, Gear = 63

Pitch circle diameter of gear, d2 = 252 mm,

Pitch circle diameter of pinion = m x z1 = 4 x 21 = 84 mm

Face width = 40 mm

Tabulate the standard tooth proportions of Involute spur gear Table.12.3/Pg.229 or Table 12.4 (a) /Pg.230

Q2.In a spur gear arrangement a pinion made of cast steel is rotating at 900 rpm and is driving a cast iron gear at 150 rpm. The teeth are to have 20 deg stub involute profile and the maximum power to be transmitted is 25 kW. Determine the module and face width. Find dynamic and wear load also. The pinion has 16 teeth with a surface hardness of 250 BHN. The allowable static stresses for pinion and gear are 103 and 55 MPa respectively. Take modulus of elasticity of pinion and gear are 207 and 96 GPa respectively.

Given Data

No of teeth of pinion, z1 = 16, n1 = 900 rpm, Power, P = 25 kW, n2 = 150 rpm. α = pressure angle = 20 degree stub involute, allowable static stresses for pinion σd1 = 103 MPa = 103 N/mm2, allowable static stresses for gear σd2 = 55MPa = 55 N/mm2

Speed ratio

z1 = 16, z2 = number of teeth on gear = i x  z1 = 6x 16 = 96

- Identifying the weaker part for design (pinion/gear)

Since the material used for pinion and gear are different then the strength factor (σd y) is used to determine the weaker part where y is the Lewis form factor

For 20 degree stub teeth Lewis form factor

Eqn. 12.5 (e) /Pg. 204

y1 = 0.1156 for pinion with number of teeth (z) of 16

y2= 0.1651 for gear with number of teeth (z) of 96

σd1 y1 = 103 x 0.1156 = 11.91 N/mm2, σd2 y2 = 55 x 0.1651= 9.08 N/mm2.

Since σd2 y2< σd1 y1 gear is the weaker and the design is based on gear.

- Module

Since diameter are unknown

m=                                                                           Eqn. 12.5(b)/Pg.204

Where Mt = transmitted torque = Rated torque x Service factor, Cs

Rated torque in N-mm

T=9.55×106×Pn2=9.55×106×25150=1591.67×103  Nmm

Transmitted torque

Mt    = Rated torque, T x Cs =  1591.67×103x 1.25=1989.58 ×103 Nmm

σd =σd2 = 55 N/mm2

Assume, k = b/m =10, Cv = 0.5, Y = πy2 = 0.1651 π, z = z2 = 96

m=2MtσdCvkYz1/3=2×1989.58×10355×0.5×10×0.1651×π×961/3=6.51 mm

Table.12.2/Pg.229

m = module = 7mm = d/z

Pitch circle diameter of pinion, d1 = m x z1 = 7 x 16 = 112 mm

Pitch circle diameter of gear, d2 = m x z2 = 7 x 96 = 672 mm

Face width = 10 m = 10 x 7= 70 mm

Transmitted load, Ft = 2Mt/d2

Ft=2×1989.58×103672=5921.42 N

Velocity, v=πd2n260000=π672×15060000=5.28m/s

For ordinary cut gears running with a velocity up to 8 m/s the velocity factor Cv

Cv=3.053.05+v=3.053.05+5.28=0.366

Eqn. 12.6(a)/Pg.205

Allowable stress

σdCv=σd2Cv=55×0.366=20.13Nmm2

Induced stress

Where b = face width = k x m = 10 x 7 = 70 mm, y = form factor =y2 = 0.1651

p= circular pitch = π x m = 7π mm

σinduced=Ftbyp=5921.470×0.1651×7π=23.3 Nmm2

Since the induced stress >allowable stress, the design is not safe from the point of view of strength.  Hence the module may increase to 8mm

m = module = 8mm = d/z

Pitch circle diameter of pinion, d1 = m x z1 = 8 x 16 = 128 mm

Pitch circle diameter of gear, d2 = m x z2 = 8 x 96 = 768 mm

Face width = 10 m = 10 x 8 = 80 mm

Transmitted load, Ft = 2Mt/d2

Ft=2×1989.58×103768=5181.2 N

Velocity, v=πd2n260000=π768×15060000=6.03m/s

For ordinary cut gears running with a velocity up to 8 m/s the velocity factor Cv

Cv=3.053.05+v=3.053.05+6.03=0.336

Allowable stress

σdCv=σd2Cv=55×0.336=18.48Nmm2

Induced stress

where b = face width = k x m = 10 x 8 = 80 mm, y = form factor =y2 = 0.1651

p= circular pitch = π x m = π x 8 mm

σinduced=Ftbyp=5181.280×0.1651×8π=15.61 Nmm2

Since the induced stress <allowable stress, the design is safe from the point of view of strength.

- Face width, b = 80 mm

- Dynamic tooth load (Fd)

Fd=Ft+K3vCb+FtK3v+Cb+Ft

K3 = 20.67                                                                                                 Page 207

C = Dynamic factor depending upon machining errors

C=ek11E1+1E2

k1 = 8.70 for 20o stub teeth                                                                        Page 207

For the velocity of 6.03 m/s the allowable error in action, e ~ 0.059

Table 12.14/Pg.237 provides the value of permissible error in action for various velocities. Fig 12.5/Pg. 209 also gives the values of permissible error in tooth profiles for quiet operation of gears for various pitch line velocities.

Where E1 and E2 are the modulus of elasticity of pinion and gear respectively.

E1 = 207 GPa = 207 x 103 MPa = 207 x 103 N/mm2

E2= 96 GPa = 96 x 103 MPa = 96 x 103 N/mm2

C=0.0598.71207×103+196×103=444.77 N/mm

Fd=Ft+K3vCb+FtK3v+Cb+Ft=5181.2+20.67×6.03×444.77×80+5181.220.67×6.03+444.77×80+5181.2=20.74 kN

Suppose that the values of modulus of elasticity may not be given. Then select the values from the Data Book. Table I.17 to 18/Pg. 472-475

OR

Table 12.14/Pg.237 provides the value of permissible error in action for various velocities. Then refer the Table 12.12/Pg.236 for values of Dynamic factor ‘C’ for different combination of materials (pinion and gear), tooth form &error in action (mm)

- Wear Load(Fw)

Fw=d1bQK≥Fd

d1 = Diameter of pinion =128 mm, K = Load stress factor

K=σes2  sinα1.41E1+1E2

Table.12.16 and 12.17/Pg.239 &240

For Cast steel pinion which is given as 250 BHN and Cast iron gear (say BHN. 180), the endurance limit σes = 617.8 MPa = 617.8 N/mm2

Table.12.16 and 12.17/Pg.239 &240

K=617.82  sin201.41207×103+196×103=1.42

Q = Ratio factorEqn.12.15 (c)/Pg.208

Q=2d2d2+d1=2z2z2+z1=2×9696+16=1.714

Fw=d1bQK=128×80×1.714×1.42=24923 N=24.923 kN

Fw = 24.923 kN > Fd (= 20.74kN). Hence the design is safe against wear

Tabulate the standard tooth proportions of Involute spur gear     Table.12.3/Pg.229 or Table 12.4 (a) /Pg.230

Q3. Design a spur gear drive required to transmit 55 kW with light shocks for 8 to 10 hours per day at 800 rpm of the pinion. The speed ratio is to be 3.2:1. The teeth are to be 20o full depth involute

Given Data

Power, P = 55 kW,n1 = 800 rpm, α = pressure angle = 20 degree full depth involute

Speed ratio ,i=n1n2=d2d1=z2z1

The speed of the gear,n2 = n1/i = 800/3.2 = 250 rpm

Material Properties

Since the materials for pinion and the gear are not specified let us assume that both are made from cast steel 0.20 % heat treated having design static stress σd1= σd2 = 193.2 MPa = 193.2 N/mm2 and BHN1=BHN2  =250

Identifying the weaker part ( pinion/gear)

Since the pinion and the gear are made of the same material, the pinion is weaker and the design is based on the pinion.

- Module

Since diameter are unknown

m =                                                                      Eqn. 12.5(b)/Pg.204

Where Mt = transmitted torque = Rated torque x Service factor, Cs

Rated torque in N-mmT=9.55×106×Pn1=9.55×106×55800=656.56×103  Nmm

Mt = transmitted torque = Rated torque in N-mm x Service factor Cs

Cs for light shocks for 8 to 10 hours per day = 1.25                 Table.12.8/Pg.235

Mt    = Rated torque, T x Cs =  656.56×103x 1.25=820.7 ×103 Nmm

- Assume k =b/m = 10

- Assume Cv = 0.5

- Assume z1 =20. z2 = 20 x 3.2 = 64

Lewis form factor y1=0.154-0.912z1=0.154-0.91220=0.1084

Form factor Y1 = π x 0.1084 and z= z1 = 20

m=2MtσdCvkYz1/3=2×820.7×103193.2×0.5×10×0.1084×π×201/3=6.27 mm

Take the standard value of module, m =7 mm                         Table.12.2/Pg.229

Pitch circle diameter of pinion, d1 = m x z1 = 7 x 20 = 140 mm

Pitch circle diameter of gear, d2 = m x z2 = 7 x 64 = 448 mm

Face width = 10 m = 10 x 7 = 70 mm

Transmitted load, Ft = 2Mt/d1

Ft=2×820.7×103140=11724.3 N

v=πd1n160000=π140×80060000=5.86 m/s

For ordinary cut gears running with a velocity up to 8 m/s the velocity factor Cv

Cv=3.053.05+v=3.053.05+5.86=0.342

Allowable stress

σallowable stress=σd×Cv=193.2×0.342=66.07 Nmm2

Induced stress

σinduced=Ftbyp=11724.370×0.1084×7π=70.26 Nmm2

Since the induced stress >allowable stress, the design is not safe from the point of view of strength. Hence the module may increase to 8mm

Take the standard value of module, m =8 mm                         Table.12.2/Pg.229

Pitch circle diameter of pinion, d1 = m x z1 = 8 x 20 = 160 mm

Pitch circle diameter of gear, d2 = m x z2 = 8 x 64 = 512 mm

Face width = 10 m = 10 x 8 = 80 mm

Transmitted load, Ft = 2Mt/d1

Ft=2×820.7×103160=10258.75 N

v=πd1n160000=π160×80060000=6.7m/s

For ordinary cut gears running with a velocity up to 8 m/s the velocity factor Cv

Cv=3.053.05+v=3.053.05+6.7=0.313

Allowable stress

σallowable stress=σd×Cv=193.2×0.313=60.47 Nmm2

Induced stress

σinduced=Ftbyp=10258.7580×0.1084×8π=47.1 Nmm2

Since the induced stress < allowable stress, the design is safe from the point of view of strength

- Face width, b = 80 mm

- Dynamic tooth load (Fd)

Fd=Ft+K3vCb+FtK3v+Cb+Ft

K3 = 20.67                                                  Page 207

C = Dynamic factor depending upon machining errors

As per the Table 12.14/Pg.237 the maximum allowable or permissible error in action for velocities 6 m/s and 8 m/s are tabulated as 0.0590 and 0.0500 mm respectively. Hence for a velocity of 6.7 m/s the error may be approximated as 0.056. The value of dynamic factor ‘C’ for the material combinations steel & steel for both pinion and gear with 20 o FDI for an error0. 0.06 mm = 686.7 N/mm. C for an error of 0.056 = 0.056 x 686.7/0.06 = 641 N/mm.

.Fd=10258.75+20.67×6.7641×80+10258.7520.67×6.7+641×80+10258.75=32306 N

Fd = 32.306 kN

- Endurance strength of gear  (Fen

Fen=σenbYm   		                       Eqn.12.13 (a)/Pg.207

Steel @ BHN = 250, σen = 429 MPa = 429 N/mm2

Fen=429×80×π×0.1084×8=93.5 kN

Since Fen> Fd the design is safe

- Wear Load(Fw)

Fw=d1bQK≥Fd

- Diameter of pinion, d1 = 160 mm

- Face width, b                 = 80 mm

- Ratio factor Q=2d2d2+d1=2×512512+160=1.524

- For pressure angle , α = 20 deg and steel – steel combination of BHN = 250, K = 0.902                                                             Table.12.16/Pg.239

Fw=160×80×1.524×0.902=17595.5 N

Fw = 17.6 kN < Fd (32.306 kN). Since Fw < Fd, the pinion may subjected to rapid wear and the surface has to be hardened to higher BHN

The condition for safe design is Fw≥Fd=32.306 kN( 32306 N)

Fw=d1bQK=160×80×1.524×K≥32306

K≥1.656

Therefore for pressure angle , α = 20 deg and steel – steel combination and  K≥1.656 BHN of pinion = 350 = BHN of gear giving the value of K = 1.893

Tabulate the standard tooth proportions of Involute spur gear. Table.12.3/Pg.229 or Table 12.4 (a) /Pg.230

# Type – II Problems: diameters or center distance or velocity given

Q4. Design a pair of spur gear to transmit 20 kW at 1200 rpm of C30 heat treated pinion 200 mm diameter to a 0.3 % heat treated forged steel gear to run at 400 rpm. The teeth are     1412 degree involute form.

Given

Power = 20 kW, n1 = 1200 rpm, n2 = 400 rpm, α = pressure angle = 1412 degree

d1= 200 mm

Pinion: C30 heat treated   steel, σd1= 220.6 MPa

Gear: 0.3 % heat treated forged steel cast steel, σd2= 138.3 MPa                                                                                                     Table 12.7/Pg.234

Speed ratio i=n1n2=d2d1=z2z1=3

Pitch circle diameter of gear, d2 = i x 200 = 3 x 200 = 600 mm

- Identifying the weaker part for design (pinion/gear)

It depends upon the product of σd y, where σd = allowable static stress and y = Lewis form factor (which depend on the number of teeth). Since the number of teeth is not given, temporarily assume z1 = 20 and z2 = i x z1 = 3 x 20 = 60.

For 1412 degree involute form

y=0.124 -0.684z

y1=0.124 -0.684z1=0.124-0.68420=0.0898

y2=0.124 -0.684z2=0.124-0.68460=0.1126

σd1×y1=220.6×0.0898=19.81Nmm2

σd2×y2=220×0.1126=24.77Nmm2

Since the capacity of pinion is less, pinion is weaker and the design is based on pinion.

Since the diameter is known

Module m=FtσdCvkY                                                      Eqn 12.5 (b)/Pg.204

Ft = Tangential tooth load Ft=1000PCsvwhere P = Power in kW, Cs = Service factor (Table 12.8/Pg.235) and v = pitch line velocity in m/s.

v=πdn1000×60=πd1n160000=π×200×120060000=12.57m/s

Assume medium shocks, 10 hours/day service Cs = 1.5

Ft=1000PCsv=1000×20×1.512.57=2386.6 N

For carefully cut gears running with a velocity up to 13 m/s the velocity factor Cv

Cv=4.584.58+v=4.584.58+12.57=0.267

σd =σd1, k = b/m = 10 (assume) and Form factor,Y = Y1 = πy1

y1=0.124 -0.684z1=0.124-0.684d1m=0.124-0.684200m=0.124-3.42×10-3m

Y=Y1=π×y1=π0.124-3.42×10-3m=0.39-0.0107m

- Module, m

m=FtσdCvkY

m2=Ftσd1CvkY1=6631.3220.6×0.267×100.39-0.018m

229.71 m2-10.60m3=2386.6

10.6m3-229.71m2+2386.6=0

m3-21.67m2+225.15=0

Solving the smaller value of module m = 3.523 mm.

Take m = 4 mm

Number of teeth on the pinion z1=d1m=2004=50

Number of teeth on the gear z2=d2m=6004=150

Face width = 10 x m = 10 x 4 = 40 mm

y1=0.124 -0.684z1=0.124-0.68450=0.1103

Circular pitch, p = π x m = 4 π mm

Allowable stress

σallowable stress=σd1×Cv=220.6×0.267=58.9Nmm2

Induced stress

σinduced=Ftbyp=2386.640×0.1103×4π=43.05 Mmm2

Since the induced stress < allowable stress, the design is safe from the point of view of strength

- Dynamic tooth load (Fd)

Fd=Ft+K3vCb+FtK3v+Cb+Ft

K3 = 20.67

@ Velocity, v = 12 m/s, the permissible error , e = 0.0330       ( Table 2.14/Pg.237)

@ Velocity, v = 12 .57 m/s, the permissible error, e ~ 0.0315. For α = 14.5 deg steel – steel combination, the dynamic factor, C =330.9 N/mm for an error of 0.03 mm.

For an error of 0.0315, C = 330.9 x 0.0315/0.03 = 347.5 N/mm

Fd=2386.6+20.67×12.57347.5×40+2386.620.67×12.57+347.5×40+2386.6=13308.6 N=13.31 kN

- Endurance strength of gear  (Fen)

Fen=σenbYm   		                       Eqn.12.13 (a)/Pg.207

For steel @BHN = 300, the endurance limit σen = 515 MPa = 515 N/mm2.

Fen=σenbYm=515×40×π×0.1103×4=28553.06 N=28.553 kN

Since Fen>Fd, the material is safe against the static load.

- Wear Load(Fw)

Fw=d1bQK≥Fd

- Diameter of pinion, d1 = 200 mm

- Face width, b                 = 40 mm

- Ratio factor Q=2d2d2+d1=2×600200+600=1.5

- For ‘α’ =  14.5 deg and steel – steel combination having BHN1 =300 and BHN2 = 300, the value of load stress factor, K = 0.990

- Fw=d1bQK=200×40×1.5×0.990=11880 N=11.88 kN≤Fd

For safe design Fw≥Fd

d1bQK≥Fd

200×40×1.5×K≥13308.6

K≥1.109

- For ‘α’ =  14.5 deg and steel – steel combination K > 1.109 having BHN1 =350 and BHN2 = 300.                                                    Table 12.16/Pg.  239

Tabulate the standard tooth proportions of Involute spur gear Table.12.3/Pg.229 or Table 12.4 (a) /Pg.230

Q5.It is required to transmit 25 kW of power from a shaft running at 1000 rpm to a parallel shaft with speed reduction 2.5:1.The center to center distance of the shaft is to be 300 mm. The material used for pinion and gear is steel & CI respectively. Considering class – II gear with tooth profile 20 degree FDI design the gear and check for dynamic and wear load. Design static stress for pinion = 200MPa and for gear 180MPa. Hardness for pinion & gear is 250 and 200 BHN respectively.

Given

Power = 25 kW , velocity ratio =2.5,Centre  distance = 300 mm

Pinion: Steel, Design static stress σd1 = 200 MPa = 200 N/mm2

Speed =n1= 1000 rpm, BHN1= 250.

Gear: Cast iron, Design static stress σd2 = 180 MPa = 180 N/mm2

Class –II gear

Velocity ratio i=n1n2=d2d1=z2z1=2.5

Speed of gear, n2

n2=n1i=10002.5=400rpm

Centre distance,a =300 =d1+d22

d1+d2=600

d2=2.5d1

3.5 d1=600

d1=171.43 mm  &d2=d1×2.5 =428.57 mm

Round –off ‘d’

let

d1=176 mm

d2=2.5 ×176=440 m

Velocity ratio i=n1n2=d2d1=440176=2.5

New centre distance a=d1+d22=176+4402=308 mm

- Identifying the weaker part for design (pinion/gear)

It depends upon the product of σdx y, where σd = allowable static stress and y = Lewis form factor (which depend on the number of teeth). Since the number of teeth is not given, temporarily assume z1 = 20 and z2 = i x z1 = 2.5 x 20 = 50 teeth

- For α = 20 degree FDI

y=0.154-0.912z

Lewis form factor y1=0.154-0.912z1=0.154-0.91220=0.1084

y2=0.154-0.91250=0.154-0.91250=0.1357

σd1×y1=200×0.1084=21.68Nmm2

σd2×y2=180×0.1357=24.44Nmm2

Since σd1y1<σd2y2, the pinion is weaker and the design is based on pinion

Since the diameter is known

Module m=FtσdCvkY                                                      Eqn 12.5 (b)/Pg.204

Ft = Tangential tooth load Ft=1000PCsvwhere P = Power in kW,    Cs = Service factor (Table 12.8/Pg.235) and v = pitch line velocity in m/s.

v=πdn1000×60=πd1n160000=π×176×100060000=9.215 m/s

Assume medium shocks, 10 hours/day service Cs = 1.5

Ft=1000PCsv=1000×25×1.59.215=4069.45 N

For carefully cut gears running with a velocity up to 13 m/s the velocity factor Cv

Cv=4.584.58+v=4.584.58+9.215=0.332

Y=Y1=πy1

y1=0.154 -0.912z1=0.154-0.912d1m=0.154-0.912176m

=0.154-5.182×10-3m

Y1=π×y1=π0.154-5.182×10-3m=0.484-0.0163m

- Module, m

m=FtσdCvkY

m2=Ftσd1CvkY1=4069.45200×0.332×100.484-0.0163 m

321.38 m2-10.82 m3=4069.45

10.92 m3-321.38 m2+4069.45=0

m3-29.43 m2+372.66=0

Solving the smaller value of module m = 3.8142mm.

Take m = 4mm

Number of teeth on the pinion z1=d1m=1764=44

Number of teeth on the gear z2=d2m=4404=110

Check for the speed ratio i=z2z1=11044=2.5

Face width = 10 x m= 10 x 4= 40 mm

Lewis form factor

y1=0.154 -0.912z1=0.154-0.91244=0.133

Circular pitch p=π×m=4π mm

Allowable stress σallowable stress=σd1×Cv=200×0.332=66.4Nmm2

Induced stress σinduced=Ftbyp=4069.4540×0.133×4π=60.87 Mmm2

Since the induced stress is less than the allowable stress, the design is safe.

- Dynamic tooth load (Fd)

Fd=Ft+K3vCb+FtK3v+Cb+Ft

K3 = 20.67

For class –II gears maximum of probable error in action for module 4

e = 0.0254                                                                                    Table 12.13/pg.237

Dynamic factor, C

For steel and CI combination & α = 20 deg FDI, @e =0.02, C = 157.3 N/mm

C for e = 0.0254 = 157.3 x 0.0254/0.02 = 199.97 N/mm.

Fd=4069.45+20.67×9.215199.97×40+4069.4520.67×9.215+199.97×40+4069.45=11723 N=11.73 kN

- Endurance strength of gear  (Fen)

Fen=σenbYm   		                       Eqn.12.13 (a)/Pg.207

For steel @BHN1 = 250, the endurance limit σen = 429 MPa = 429 N/mm2.

Fen=σenbYm=429×40×π×0.133×4=28680 N=28.68 kN

Since Fen>Fd, the material is safe against the static load

- Wear Load(Fw)

Fw=d1bQK≥Fd

- Diameter of pinion, d1 = 176 mm

- Face width, b                 = 40 mm

- Ratio factor Q=2d2d2+d1=2×440176+440=1.429

- For ‘α’ =  20 deg and steel – CI combination having BHN1 =250 and BHN2 = 200, the value of load stress factor, K = 1.344 (max BHN2 = 180)

- Fw=d1bQK=176×40×1.429×1.344=13521 N=13.521 kN≥Fd

For safe designFw≥Fd. Hence the material is safe against wear.

Tabulate the standard tooth proportions of Involute spur gear     Table.12.3/Pg.229 or Table 12.4 (a) /Pg.230

Q6. A cast steel pinion rotating at 900 rpm is to drive a cast iron gear at 144 rpm. The teeth are to standard 20 deg sub involute and the maximum power to be transmitted is 25 kW. Design the gear. Design static stress for pinion = 103 MPa and for gear 55 MPa. The pinion is surface hardened to BHN 250

Given

Pinion: Cast steel, Design static stress σd1 = 103 MPa = 103 N/mm2

Speed =n1= 900 rpm. Gear: Cast iron, Design static stress σd2 = 55 MPa = 55 N/mm2.Speed =n2= 900 rpm. The maximum power to be transmitted is 25 kW. Hardness of pinion = 250 BHN

The diameters are unknown. The number of teeth will be assumed selecting not less than 14 for 20 deg stub tooth                                           Table 12.4 (b)/Pg. 230

The minimum number of teeth on pinion may be assumed as 16 in order to satisfy the required transmission ratio of 900/144 = 6.25

Number of teeth on pinion, z1 =16

Number of teeth on gear, z2 = (900 x 16)/144 = 100.

Lewis form factor, y=0.175-0.95z for 20 deg stub teeth system

Eqn12.5 (e)/Pg. 204

y1for z1 of 16 = 0.116, y2 for z2 of 100 = 0.166 or can select the value of Lewis form factor, y from the Table 12.5/Pg.232 or can calculate from the equation given from DHB.

- Identifying the weaker part for design (pinion/gear)

σd1×y1=103×0.116=11.95Nmm2

σd2×y2=55×0.166=9.13Nmm2

Since σd2y2<σd1y1  , the gear is weaker and hence the design is based on the gear.

Do not use those values for design as they are based on temprerory number of teeth on pinion and gear

- Module

- Since diameter are unknown

m=2MtσdCvkYz1/3

Since the maximum power transmitted is given, the maximum torque can be determined

The maximum power to be transmitted is 25 kW

The maximum torque to be transmitted = Mt

Mt=9.55×106×Pmaxn2=9.55×106×25144=1657986 N mm

m=2MtσdCvkYz1/3

σd1 = σd2 = 55 N/mm2

- Assume k =b/m = 10

- Assume Cv = 0.5

Y = Form factor = Y = π x y2

Lewis form factor, y=0.175-0.95z  for 20 deg stub teeth system

Eqn 12.5(e)/Pg. 204

z=z2= 100

m=2MtσdCvkYz1/3=2×165798655×0.5×10×π×0.166×100=6.14 mm

The preferred series of module (mm) given in Table 12.2 /Pg.229 is 8 or 7 mm (choice 2). Let select the second choice, m = 7 mm

Take the second choice of the value of module, m =7 mm      Table.12.2/Pg.229

Pitch circle diameter of pinion, d1 = m x z1 = 7 x 16 = 112 mm

Pitch circle diameter of gear, d2 = m x z2 = 7 x 100 = 700 mm

Face width = 10 m = 10 x 7 = 70 mm

- Transmitted load, Ft = 2Mt/d1

Ft=2×1657986700=4737.10N

v=πd2n260000=π700×14460000=5.28m/s

For ordinary cut gears running with a velocity up to 8 m/s the velocity factor Cv

Cv=3.053.05+v=3.053.05+5.28=0.366

Allowable stress

σallowable stress=σd×Cv=55×0.366=20.14Nmm2

Induced stress

σinduced=Ftbyp=4737.1070×0.166×7π=18.54 Mmm2

Since the induced stress < allowable stress, the design is safe from the point of view of strength

- Face widthb = 70 mm

The pinion is surface hardened to BHN 250

- Wear Load(Fw)

Fw=d1bQK≥Fd

- Diameter of pinion, d1 = 112 mm

- Face width, b                 = 70 mm

- Ratio factor Q=2d2d2+d1=2×700112+700=1.724

Pinion steel with BHN 250 and the gear CI, the value of load stress factor for 20 deg pressure angle = 1.344                                                          Table 12.16/pg.239

Fw=112×70×1.724×1.344=18165.72 N=18.17 kN

- Dynamic tooth load (Fd)

Fd=Ft+K3vCb+FtK3v+Cb+Ft

C = Dynamic factor depending upon machining errors

As per the Table 12.14/Pg.237 the maximum allowable or permissible error in action for velocities 5 m/s and 6 m/s are tabulated as 0.0640 and 0.0590 mm respectively. Hence for a velocity of 5.28 m/s the error may be approximated as 0.063. The value of dynamic factor ‘C’ for the material combinations of CI &steel with 20 deg stub tooth form for an error 0. 0.06 mm = 488.4 N/mm. C for an error of 0.063 = 0.063 x 488.4/0.06 = 512.4 N/mm.

Fd=Ft+K3vCb+FtK3v+Cb+Ft=4737.1+20.67×5.28512.4×70+4737.120.67×5.28+512.4×70+4737.1=19002.7 N

Fd= 19002.7 N =19 kN. Since Fw < Fd, the pinion may subjected to rapid wear

Select a carefully cut gear.Fig.12.4/Pg.209 for a module m = 7 mm, the probable error is ~ 0.033 mm.The value of dynamic factor ‘C’ for the material combinations of CI &steel with 20 ostubtooth form for an error 0. 0.03 mm = 244.2 N/mm for an error of 0.033 = 0.033 x 244.2/0.03= 268.6 N/mm. Recalculate the dynamic load

Fd=Ft+K3vCb+FtK3v+Cb+Ft=4737.1+20.67×5.28268.6×70+4737.120.67×5.28+268.6×70+4737.1=14521.6 N

Fd= 14521.6 N =14.52 kN which is less than the wear load Fw=18.17 kNFw the design is safe from the standpoint of wear.

Tabulate the standard tooth proportions of Involute spur gear Table.12.3/Pg.229 or Table 12.4 (a) /Pg.230