Design Procedure for Riveted Boiler Joints

Longitudinal Joint: Butt Joint

- Type of Joint: If the type of joint is not given, select the type of joint from Table 5.2/Pg.84 based on the diameter of the shell.

- Determine the thickness of shell joint

t=piD2σt η+1 mm

Eqn5.1/Pg.79 with a modification of corrosion allowance of 1mm

If the efficiency is not specified, the efficiency may be obtained from Table 5.1/Pg.84

Where pi = Pressure acting inside the boiler, N/mm2

D= Internal diameter of pressure vessel, mm

σt = Allowable tensile stress of shell plate , N/mm2

η = Efficiency of longitudinal joint

Recommended type of Joints based on D and t

The standard value of plate thickness is to be selected from 	the Table 5.3(c)/Pg.85

- Determine the diameter ,d, of the rivet hole from Unwin’s formula

- If the thickness of the plate is more than 8mm (t > 8 mm) the diameter of rivet hole is obtained from Unwin’s formula

d=6.07t  		                 Eqn5.11 (d) /Pg.81

- If   t≤8mm, diameter should be calculated by equating the shearing resistance of the rivets to the crushing resistance

Ps=Pc

n1+1.875n2πd24τ=n1ti+n2tdσc    Eqn5.6 (d&e) /Pg.80

d=4n1ti+1.875n2tπn1+1.875n2×σcτ            Eqn 5.10(a) /Pg.81

For   lap joint ti =t and n2 =0

For equal cover butt joint n1 =0

Select the diameter of rivet, dr, from Table.5.3 (b) page 84

Or

d = dr + 1      for 12 mm < d < 24 mm

d = dr + 1.5   for 24 mm < d < 36 mm

d = dr + 2      for d > 36 mm

Note: Use diameter of rivet hole for all calculations

- Determine the pitch of rivets

- The pitch of the rivets is obtained by equating the shearing resistance of the rivets to the tearing resistance of the plate

Ps=Pt

n1+1.875n2πd24τ=p-dtσt              Eqn5.6 (d&b) /Pg.80

Optimum pitch

p=n1+1.875n2πd2τ4tσt+d         Eqn 5.12(a) /Pg.81

For   lap joint  n2 =0

For equal cover butt joint n1 =0

- Maximum pitch, pmax=k1t+41 mm		                         Eqn5.12 (b) /Pg.81

Where k1 = a constant Table 5.4(a)/Pg.85

Adopt lower value of pitch, p.

- Determine the transverse pitch ( distance between rows of rivets

It can be calculated from the appropriate equations 5.14 (a) to (d) /Pg.82 & Eqn 5.15 (a &b)/Pg.82 and 83

- Determine the diagonal pitch for staggered joint

pd≥p+d2                                        Eqn5.17 (a) /Pg.83

pd=2.25d to 2.5 d                                                       Eqn5.17 (b) /Pg.83

- Determine the thickness of the cover straps from the appropriate equations given in Eqns 5.4 (a to f)/Pg.79 & 80

8.    Determine the margin, m = 1.5 d		                                                                                      Eqn5.16 /Pg.83

- Efficiency

η=least value of Pt,Ps and PcP

The above procedure may also be used for ordinary riveted joint

Circumferential Joint: Lap Joint

1.	The thickness of shell plate, diameters of rivet and rivet holes and margin are same for both longitudinal and circumferential joint.

2.	Total steam load P=π4D2pi

3.	Strength of each rivet

In shear	         Ps=π4d2τ

In crushing	        Pc=dtσc

4	No of rivets required

N=Total steam load PMiminum strength of rivetPsorPc

5.	Pitch of rivets

η=p-dp                                                           Eqn5.9 (e) /Pg.83

Assume suitable value of efficiency                                           Table 5.1/Pg.84

Or

Optimum pitch

p=n1+1.875n2πd2τ4tσt+d                       Eqn 5.12(a) /Pg.81

For   lap joint  n2 =0

Maximum pitch, pmax =k1 t+41 mm 	                              Eqn5.1 (b) /Pg.81

Where k1 = a constant (Table 5.4(a)/Pg.85

6.	     Number of rivets in one row, n

The circumferential length considering the boiler diameter, D and plate thickness is obtained as

circumferential length =πD+t

Considering the pitch and the number of rivets per row, n

circumferential length =np

n=πD+tp

7.	 Number of rows

Number of rows=Nn

After determining the number of rows the type of joints such as single riveted or double riveted lap joint is selected. Adjust the number of rivets in each row and the pitch of rivets.

8.	Determine the transverse pitch, pt (distance between two rows of rivets)

9.	Determine the overlap of the plate where the longitudinal and circumferential joints meet

Overlap = (Number of rows of rivet-1) pt + 2 m

Where m = margin =1.5 d

10. Efficiency of circumferential joint

η=least value of Pt,Ps and PcP

Note

- Table 5.1/Pg. 84 gives the values of efficiency of commercial boiler joints.

- Table 5.2/Pg. 84 gives the recommended type of joints based on diameter of boiler shell.

- Table 5.3(b) /Pg. 84 gives the nominal diameter of the rivets.

- Table 5.3(c) /Pg. 85 gives the thickness of plates.

- Table 5.4(b) /Pg. 85 gives the recommended values of rivet hole diameter, pitch and efficiency.

- Pressure vessels are designed based on a factor of safety of 5 as given under the note section of Table 5.6 /Pg. 85.

Q1

Design a double riveted longitudinal joint for a boiler shell whose inner diameter is 1600 mm and steam pressure is 2 MPa. The ultimate tensile, shear and crushing strength are 450,400 and 800 MPa respectively.

Solution

D = 1600 mm, pi = 2 MPa = 2 N/mm2, σu=450 MPa ,  τu=400 MPa, σuc=800 MPa

Since the allowable stresses are not given, take a factor of safety of 5 to obtain the values.

σt=σu5=4505=90 MPa , τ=τu5=4005=80 MPa , σc=σuc5= 8005=160 MPa

Since the efficiency of the joint is not provided, from Table 5.2/Pg.84 the recommended joint for pressure vessel with internal diameter 1600 mm the type of butt joint may be double or triple riveted joint. Selecting the double riveted joint, its efficiency range is 72 – 82 %. Take the efficiency = 80 %.

Table 5.1/Pg.84.

- Thickness of the shell

t=piD2σt η+1 mm                                            Eqn5.1/Pg.79

=2×16002×90×0.8+1=23.22 mm

From Table 5.3(c)/Pg.85, the standard value of plate thickness is 25 mm.  t = 25 mm

- Diameter of rivet hole

Since the thickness is more than 8 mm, the diameter of rivet hole can be calculated from the Unwin’s formula,  d=6.07t                                                        Eqn5.11 (d)/Pg.79

d=6.07t=6.0725=30.35 mm

From Table 5.3 (b)/Pg. 84, the nominal diameter of rivet (dr) is 30 mm.

d= dr + 1.5   for 24 mm < d < 36mm, dr = 30

Diameter of rivet hole = 30 +1.5 = 31.5 mm

- Pitch of rivets

The longitudinal joint of a boiler is a double riveted butt joint with two equal or unequal cover straps. Assuming two equal cover straps with equal width and thickness, the remaining computations can be made.

Optimum pitch

p=n1+1.875n2πd2τ4tσt+d              Eqn 5.12(a) /Pg.81

For equal cover butt joint n1 =0

p=1.875n2πd2τ4tσt+d=1.875×2π31.52×804×25×90+31.5=135.41 mm

- Maximum pitch, pmax=k1t+41 mm		                 Eqn5.12 (b) /Pg.81

Where k1 = a constant (Table 5.4(a)/Pg.85

From Table 5.1/Pg. 84, for double row butt joint with two cover plates, No.of rivets per pitch = 2. From Table 5.4(a)/Pg. 85, for the No.of rivets per pitch of 2, the Coefficient, k1 for double butt strapped joints = 3.5

pmax=k1t+41 mm=3.5×25+41 =128.5 ~128 mm

Since pmax is less than the calculated value the pitch of the joint p = 128 mm.

For leak free joint p ≤6 d

12831.5=4.06 <6                               Eqn5.12 (d)/Pg.82

Hence the joint is leak proof

- Transverse pitch (distance between the rows of rivet)

The transverse pitch for equal number of rivets in the rows and for staggered     (zig- zag) riveting

pt≥0.33p+0.67d                                                         Eqn5.14 (b)/Pg.82

pt≥0.33p+0.67d=0.33×128+0.67×31.5=63.35 mm~ 64 mm

For longitudinal joint the minimum value of transverse pitch (ASME boiler code)

pd=12831.5=4.06

If pd>4, pt=134d +0.1p-d=134×31.5+0.1128-31.5=64.78 ~ 65 mm

- Diagonal pitch for staggered joint = 2.25 d to 2.5 d

- Thickness of the cover straps

Thickness of double butt strap of equal width having ordinary riveting Eqn5. 4 (c)/Pg.80

ti=t0=0.625 t=0.625×25=15.62~16 mm

- Margin, m

m=1.5 d=1.5×31.5=47.25 mm

- Efficiency

η=least value of Pt,Ps and PcP

- Tearing efficiency ηt=p-dp=128-31.5128=75.4 %                         Eqn5.9 (e)/Pg.81

- Shearing or rivet efficiency ηs=π4d2n1+1.875n2τptσt                          Eqn5.9 (f)/Pg.81

ηs=π4×31.520+1.875×280128×25×90=81.18%

- Crushing efficiency

ηc=PcP

Where Pc = Resistance to crushing of rivets

Pc=n1ti+n2tdσc                                     Eqn5.6 (e)/Pg.80

ηc=n1ti+n2tdσcptσt=0+2×2531.5×160128×25×90=87.5 %

The efficiency is the least = 75.4 %

Q2. Design circumferential joint for the above problem

The circumferential joint is always a lap joint

- Type of joint: Double riveted lap joint with zig – zag riveting. Number of rivets = 2.

- The thickness of shell plate, diameters of rivet and rivet holes and margin are same for both longitudinal and circumferential joint.

- Thickness of plate = 25 mm, diameter of rivet hole = 31.5 mm, diameter of rivet = 30 mm, margin = 47. 25 mm

Number of rivets in single shear = n1 =2, number of rivets in double shear = n2 = 0.

- Number of rivets

N=Total steam load PMiminum strength of rivetPsorPc

Based on Ps

Ns= π4D2piπ4d2τ=16002×231.52×80=66

Based on Pc                                 Nc=π4D2pidtσc =π4×16002×231.5×25×160=32

Hence maximum of these values is the total number of rivets in circumferential joint.

N = 66

- Pitch

- Optimum pitch

p=n1+1.875n2πd2τ4tσt+d                                       Eqn 5.12(a) /Pg.81

p=2+1.875×0 π×31.52×804×25×90+31.5 ~ 88 mm

- Maximum pitch, pmax=k1 t+41 mm		                             Eqn5.12 (b) /Pg.81

Where k1 = a constant                                                        Table 5.4(a)/Pg.85

k1 = 2.62 for lap joint with two rivets per pitch

pmax=2.62×25+41=106.5 mm

Standard pitch is the lower value p = 88 mm

- Number of rivets per row

n=πD+tp=π1600+2588=58

Number of rows N/n = 66/58 = 1.14

As the minimum number of possible row = 2, pitch have to be recalculated

Number of rivets per row = n = 66/2 = 33

Pitch p=πD+tn=π1600+2533=155 mm which is more than the p max.

Limiting the number of rows = 2, Increasing the total number of rivets = N = n x 2 = 58 x 2 =106, number of rivets per row = n = 58

p=πD+tn=π1600+2558=88 mm

The value of the pitch is within the range.

- Transverse pitch

The transverse pitch for equal number of rivets in the rows for staggered riveting

pt≥0.33p+0.67d                                                         Eqn5.14 (b)/Pg.82

pt≥0.33×88+0.67×31.5=50.15~ 52mm

- Overlap of the plate where the longitudinal and circumferential joints meet

Overlap = (Number of rows of rivet-1) pt + 2m = (2-1) x 52 +2 x 31.5 =115 mm

- Efficiency of circumferential joint should not be less than that of the longitudinal joint

η=least value of Pt,Ps and PcP

ηt=p-dp=88-31.588=64.2 %

- Shearing or rivet efficiency ηs=π4d2n1+1.875n2τptσt                     Eqn5.9 (f)/Pg.81

ηs=π4×31.522+1.875×08088×25×90=63%

- Crushing efficiency

ηc=PcP

Where Pc = Resistance to crushing of rivets

Pc=n1ti+n2tdσc                                     Eqn5.6 (e)/Pg.80

ηc=n1ti+n2tdσcptσt=2×16+0×2531.5×16088×25×90=81.45 %

The efficiency is the least = 63 %

Q3. A cylindrical pressure vessel with a 1.5 m inside diameter is subjected to internal steam pressure of 1.5 MPa. It is made from steel plate by triple-riveted double-strap longitudinal butt joint with equal straps. The pitch of the rivets in the outer row is twice of the pitch of the rivets in the inner rows. The rivets are arranged in a zigzag pattern. The efficiency of the riveted joint should be at least 80%. The permissible stresses for the plate and rivets in tension, shear and compression are 80, 60 and 120 N/mm2 respectively. Assume that the rivet in double shear is 1.875 times stronger than in single shear. Design the joint.Draw a neat sketch of the riveted joint showing calculated values of dimensions.

pi = Pressure acting inside the boiler, N/mm2 = 1.5

D= Internal diameter of pressure vessel, mm = 1500

σt = Allowable tensile stress of shell plate , N/mm2 = 80, Allowable shear  stress, N/mm2 = 60, Allowable compression stress, σc , N/mm2 = 120

η = Efficiency of longitudinal joint = 80 %

- Type of joint

Triple-riveted double-strap longitudinal butt joint with equal straps

- Thickness of the shell

t=piD2σt η+1 mm                                            Eqn5.1/Pg.79

=1.5×15002×80×0.8+1=18.6 mm

From Table 5.3(c)/Pg.85, the standard value of plate thickness is 20 mm.  t = 20 mm

- Diameter of rivet hole

Since the thickness is more than 8 mm, the diameter of rivet hole can be calculated from the Unwin’s formula,  d=6.07t                                                        Eqn5.11 (d)/Pg.79

d=6.07t=6.0720=27.15 mm

From Table 5.3 (b)/Pg. 84, the nominal diameter of rivet (dr) is 27 mm.

d= dr + 1.5   for 24 mm < d < 36mm

Diameter of rivet hole = 27 +1.5 = 28.5 mm

- The longitudinal joint of a boiler is a triple riveted double strap butt joint with two equal cover straps. The pitch of the rivets in the outer row is twice of the pitch of the rivets in the inner rows. The rivets are arranged in a zigzag pattern

Number of rivets per pitch

Outer row: 1, Middle row: 2, inner row: 2

As per the given conditions there are 5 rivets per pitch length and since the straps are of equal width, all the rivets are in double shear

Optimum pitch

p=n1+1.875n2πd2τ4tσt+d         Eqn 5.12(a) /Pg.81

For equal cover butt joint n1 =0

p=1.875n2πd2τ4tσt+d=0+1.875×5π28.52×604×20×80+28.5=252.8 mm

- Maximum pitch, pmax=k1t+41 mm		                 Eqn5.12 (b) /Pg.81

Where k1 = a constant (Table 5.4(a)/Pg.85

From Table 5.1/Pg. 84, for double row butt joint with two cover plates, No.of rivets per pitch = 5. From Table 5.4(a)/Pg. 85, For 5 nos of rivets per pitch, the coefficient, k1 for double butt strapped joints = 6

pmax=k1t+41 mm=6×20+41 =161

Since pmax is less than the calculated value the pitch of the joint p = 161 or 160 mm.

For leak free joint p ≤6d

16028.5=5.61 <6                               Eqn5.12 (d)/Pg.82

Hence the joint is leak proof

- Transverse pitch (distance between the rows of rivet)

The transverse pitch for joints where the longer pitch is double the shorter ones and the inner rows are staggered (zig- zag) riveted

pt≥0.2p+1.15d                                                         Eqn5.15 (a)/Pg.82

pt≥0.2×160+1.15×28.5=64.78 mm~ 65 mm

- Transverse pitch between the successive inner rows

pt≥0.165 p+0.67d                                                      Eqn5.15 (a)/Pg.82

- Thickness of the cover straps

Thickness of double butt straps of equal width having every alternate rivet in the outer rows are omitted                                                                                         Eqn5.4 (d)/Pg.82

ti=t0=0.625 p-dp-2dt=0.625×20×160-28.5160-2×28.5=15.95~16 mm

- Margin, m

m=1.5 d=1.5×28.5=42.75 mm=43 mm

- Efficiency

η=least value of Pt,Ps and PcP

- Tearing efficiency ηt=p-dp=160-28.5160=82.2 %               Eqn5.9 (e)/Pg.81

- Shearing or rivet efficiency ηs=π4d2n1+1.875n2τptσt                     Eqn5.9 (f)/Pg.81

ηs=π4×28.520+1.875×560160×20×80=140%

- Crushing efficiency

ηc=PcP

Where Pc = Resistance to crushing of rivets

Pc=n1ti+n2tdσc                                     Eqn5.6 (e)/Pg.80

ηc=n1ti+n2tdσcptσt=0+5×2028.5×120160×20×80=134 %

The efficiency is the least = 82.2 %