Strain Strain is defined as the relative change in the position of points within a body that has undergone deformation. Normal strain (εii): It is the ratio of the change in length to original length of a linear element considered in the body due to the deformation. Shear strain(γij): It is the reduction in 900 angle between two mutually perpendicular lines considered in the body due to deformation. The strain-displacement relation: A body undergoes a strain because there are relative displacements between different points in the body when it is subjected to load. If there is no relative displacement, the body undergoes rigid body motion. Consider a line AB of length dx oriented along the x-direction. Let u be the displacement component in x-direction at point A. Then the displacement of point B which is at a distance dx is u+∂u∂x dx. The change in length of AB =u+∂u∂x dx-u=∂u∂x dx ∴ the normal strain in x-direction, εxx=∂u∂x dxdx=∂u∂x; dx A u+∂u∂xdx B u. dy A v+∂v∂ydy C v Consider another line AC of length dy oriented along the y-direction. Let u be the displacement component in y-direction at point A. Then the displacement of point C which is at a distance dy is v+∂v∂y dy. The change in length of AC =v+∂v∂y dy-v=∂v∂y dy ∴ the normal strain in y-direction, εyy=∂v∂y dydy=∂v∂y. Similarly, along z-direction, let w be the displacement component in z-direction at point A. Then the displacement of point D which is at a distance dz is w+∂w∂z dz. The change in length of AD =w+∂w∂z dz-w=∂w∂z dz ∴ the normal strain in z-direction, εzz=∂w∂z dzdz=∂w∂z; dz w+∂w∂zdz A w D. Shear strain (γxy) Consider the perpendicular lines AB and AC together. We have considered the normal strains happening to these lines. In addition to the x- displacement, the line AC has displacement in the y-direction also. Because of this, the end B moves to a distance v+∂v∂xdx.; u+∂u∂ydy x v+∂v∂xdx dx dy B C A φ θ u v. So, tanθ=v+∂v∂xdx-vdx=∂v∂x . For small deformation, tanθ≅θ θ=∂v∂x Similarly, line AC not only has y-displacement, but a x-displacement u+∂u∂ydy also. ∴tanφ≅φ=u+∂u∂ydy-udy=∂u∂y The shear strain is defined as the reduction in the 90° angle between two perpendicular lines due to deforation. γxy=θ+φ=∂u∂y+∂v∂x Similarly, when we consider the perpendicular lines AC and AD (in the y-z plane) γyz=∂v∂z+∂w∂y And AB and AD (in the x-z plane) γzx=∂w∂x+∂u∂z We can notice that shear strain γxy=γyx, γyz=γzy, and γzx=γxz. We can write the strain tensor as εij=εxxεxyεxzεyxεyyεyzεzxεzyεzz Where εxy=γxy2, εyz=γyz2, and εxz=γxz2 Plane stress and Plane strain In many engineering applications, the state of stress or strain at a point can be represented in one plane (i.e., no stress or strain components in the third direction). Such a system is known as plane stress or plane strain. We will see later that both plane stress and plane stain conditions cannot exist simultaneously. ↼ σxx σxx τxy τxy ↼ σyy τxy Eg. plane stress ⇀ τxy σyy ⇀; εyy plane strain εxx γxy. ⇀ ⇀ ↼ ↼ σxxτyx0τxyσyy0000; σxx000σyy0000; σxx00000000; 0τyx0τxy00000; 0000σyyτzy0τyzσzz; σxx0τzx000τxz0σzz. bi-axial; uni-axial; ⇀ ⇀ ↼ ↼ pure shear. The problems in solid mechanics involve - Axial loading of bars; Normal stresses on cross section (Tensile or compressive). - Torsional loading of circular shafts; Shear stresses on cross section. - Bending (Flexural/Transverse) loading of beams; Normal and shear stresses. - Combined loading (Axial+bending+torsion).