Symmetric bending y Normal stresses in bending Consider a straight prismatic beam made of homogeneous material, having a cross-sectional area that is symmetrical with respect to an axis, and the bending moment is applied about an axis perpendicular to this axis of symmetry (principal axes) as shown. If the bending moment is constant, it is called pure bending (simple or symmetric bending); z x. By using a highly deformable material such as rubber, we can illustrate what happens when a straight prismatic member is subjected to a bending moment. Consider the undeformed bar which has a square cross section and is marked with longitudinal and transverse grid lines. When a bending moment is applied, it tends to distort these lines into the pattern shown. Notice that the longitudinal lines become curved and the vertical transverse lines remain straight and yet undergo a rotation. The plane cross section remain plane after bending. The bending moment causes the material within the bottom portion of the bar to stretch and the material within the top portion to compress. Consequently, between these two regions there must be a surface, called the neutral surface, in which longitudinal fibers of the material will not undergo any change in length. Consider a prismatic beam with a cross section with vertical axis of symmetry subjected to pure bending moment in the plane of symmetry. Let us consider the internal loading by cutting the beam at C.; y z x y. Consider a small elemental area dA on the cross section which is at a distance (y,z) from the y and z axes. The three stress components acting at that location can be considered as normal stress σxx and two shear stresses τxy and τxz.; Mz. From the equilibrium consideration, ΣFx=0 ≫ σxx dA=0; ΣMx=0 ≫ τxz dA y-τxy dA z=0. ΣFy=0 ≫ τxy dA=0; ΣMy=0 ≫ σxxdA z=0. ΣFz=0 ≫ τxz dA=0; ΣMz=0 ≫ -Mz-σxx dA y=0. Let the member be divided in to a large number of small cubic elements whose faces are parallel to the co-ordinate planes. When the beam is subjected to pure bending (constant bending moment M), all the faces represented in the two projections are at 90° to each other. Hence, we can conclude that γxy=γxz=0. Therefore, τxy=τxz=0. The only non-zero stress component on the cross section is σxx. Hence, at any point of a slender member subjected to pure bending, we have a state of uni-axial stress state.. Referring to the figure, for a positive bending moment M (sagging moment), the line AB will have a decrease and A’B’ will have an increase in length. So, the stress σxxand strain εxx are negative (compressive) on the upper portion and positive (tensile) on the bottom part.. Therefore, there must be a surface parallel to the upper and lower surfaces, where σxx and εxx are zero. This surface is known as neutral surface and the line of intersection of the neutral surface with the cross section is known as neutral axis. Consider the origin of co-ordinates at the neutral surface. Let ρ denotes the radius of arc DE, θ denotes the central angle corresponding to DE, and L denotes the undeformed length of DE. L=ρθ Consider the element JK situated at a distance y above the neutral surface. After bending, JK bends to an arc of length L'=(ρ-y)θ. The deformation of JK, δ=L'-L=ρ-yθ-ρθ=-yθ. The longitudinal strain on JK, εxx=δL=-yθρθ=-yρ The normal stress on the element JK, σxx=Eεxx=-Eρy σxx dA=0≫≫ -EρydA=0 -EρydA=0 ≫≫ydA=0 Ay=0≫≫ y=0 That is, the neutral axis passes through the centroid of the cross section; -Mz-σxx dA y=0≫σxxydA=-Mz -EρyydA=-Mz≫Eρy2dA=Mz y2dA=Izz, moment of inertia of the cross section of the beam w.r.t the z-axis (neutral axis). EIzzρ=Mz. ∴σxx=-Eρy=-MzIzzy This is known as flexure formula. When y=c, (top surface) the stress is maximum. ∴σxx (max ) =-MzIzzc=-MzS , S is known as the section modulus. The bending stress is maximum at the top and bottom surfaces where y is maximum. One will be a compressive stress and the other will be a tensile stress depending upon the sign of the bending moment. Even though, the results are derived for pure bending (constant bending moment and no shear force), it can be used for cases where shear forces are also present if the beam is slender (i.e., cross section is small compared to its length). Such beams are called Euler-Bernoulli beams (thin beams). Due to bending, cross sectional planes rotate and plane sections will remain plane after bending. If cross section is large compared to the length, they are called Timoshenko beams (thick beams) and, in such beams, the effect of shear deformation cannot be neglected. Here, the cross-sectional planes will not remain plane after bending as shown below. In such cases, Timoshenko beam theory has to be applied. #Example 1 A cast iron machine part is acted upon by the 3 kN.m couple as shown. Knowing that E=165 GPa and neglecting the effect of fillets, determine (a) the maximum tensile and compressive streses in the casting, (b) the radius of curvature of the casting.; Mz y x z. First, we have to locate the centroid of the cross section through which the neutral axis (z-axis) passes. Ai mm2; yi mm; Aiyi. 1.; 20×90=1800; 50; 90000. 2.; 40×30=1200; 20; 24000. ΣAi=3000; ΣAiyi=114000. y=ΣAiyiΣAi=38 mm; z. Moment of inertia of the cross-section about the neutral axis (centroidal z-axis) Izz=90×20312+1800×122+30×40312+1200×182 Izz=868×103 mm4=868×10-9 m4; z. The flexure formula is σxx=-MzIzzy Here, bending moment is negative as per the sign convention. Mz=-3 kN.m The bending stress is maximum at the extreme surfaces (y=+22 mm and y=-38 mm) σxxy=+22mm=--3×103N.m868×10-9 m422×10-3m=+76×106 Pa σxxy=-38mm=--3×103N.m868×10-9 m4-38×10-3m=-131.6×106 Pa σxx=-Eρy=-MzIzzy ≫≫1ρ=MzEIzz=-3×103N.m165×109Nm2868×10-9 m4= -20.95×10-3m-1 Negative sign shows that the curvature is convex. The radius of curvature, ρ=47.7 m #Example 2 The rectangular tube shown is extruded from an aluminium alloy for which σY=150 MPa, σU=300 MPa, E=70 GPa. Neglecting the effects of fillets, determine the maximum bending moment which can be applied considering a factor of safety of 3. Also determine the radius of curvature of the tube.; z y z y x 8 mm 120 mm 80 mm. Since, the cross-section is symmetrical to both y and z axes, the centroid is at its centre. The neutral axis is the z-axis. Izz=80×120312-64×104312=5521×103 mm4=5521×10-9 m4 The allowable stress, σallow=σUFactor of safety=300 MPa3=100 MPa <σY. The allowable stress is less than the yield stress. Therefore, the tube remains within the elastic limit. Hence, the flexure formula is applicable. The flexure formula is σxx=-MzIzzy Here, bending moment is positive as per the sign convention. Mz=M The bending stress is maximum at the extreme surfaces (y=±60 mm) 100×106Nm2=-M5521×10-9 m460×10-3m≫≫M=9201 N.m σxx=-Eρy=-MzIzzy ≫≫1ρ=MzEIzz=9201N.m70×109Nm25521×10-9 m4= +23.8×10-3m-1 Negative sign shows that the curvature is concave. The radius of curvature, ρ=42 m #Example 3 The W360 ×79 rolled-steel beam (section modulus=1270×103mm3) AC is simply supported and carries the uniformly distributed load shown. Determine the location and magnitude of the maximum normal stress due to bending.. Considering the free-body diagram of the beam, ΣFx=0 ≫≫ XA=0 ΣMA=0≫≫ RC×9m- 20kNm×6m×3m=0≫ RC=40 kN ΣFy=0 ≫≫ RC+YA-20kNm×6m=0 ≫≫YA=80 kN; XA RC YA. Section 1-1 (0≤x≤6 m) ΣFy=0 ≫≫ YA-V1-20kNm×x=0 V1=80-20x kN x=0, V1=80 kNx=6m, V1=-40 kN ΣM1-1=0 ≫≫ -YA×x+M1+20kNm×x22=0 M1=80x-10x2 kNmx=0, M1=0 x=6m, M1=120 kNm; M1 V1 x RC YA. Section 2-2 (6m≤x≤9 m) ΣFy=0 ≫≫ YA-V2-20kNm×6=0 V2=-40 kN x=6m, V2=-40 kNx=9m, V2=-40 kN ΣM2-2=0 ≫≫ -YA×x+M2+20kNm×6×x-3=0 M2=80x-120(x-3) kNmx=6m, M2=120 kNm x=9m, M2=0; V2 M2 x RC YA. The shear force becomes zero at D in section 1-1. 80-20x=0≫≫x=4 m The maximum bending moment corresponding to point D M1=80x-10x2=80×4-10×42=160 kN.m Neglecting the effect of shear deformation, the magnitude of maximum normal stress σxx=Mz maxS=160×103 N.m1270×10-6m3=126×103 Pa. Exercise 1 Two vertical forces are applied to a beam of the cross section shown. Determine the maximum tensile and compressive stresses in portion BC of the beam. Exercise 2 The beam shown is made of nylon for which the allowable stress is 24 MPa in tension and 30 MPa in compression. Determine the largest moment M that can be applied to the beam. Exercise 3 A rigid steel bar DEF is welded to the straight steel beam AB of cross-section 50 mm×50 mm. Draw the shear force and bending moment diagram of the beam AB for the load shown. Determine the maximum bending developed in the beam and its location.; 50 kN/m 160 kN 8m 3m 5m. Exercise 4 A 5m long, simply supported steel beam AD with rectangular cross-section is to carry loads as shown. The allowable normal stress for the grade of steel used is 160 MPa. If the width of the beam is 50 mm, what should be its depth h?; 50 mm h /m. Exercise 5 Select a wide flanged beam for the above instead of rectangular section.; ×103. References: - Mechanics of Materials, 2/e, J. M. Gere & S. P.Timoshenko, CBS Publishers. - Mechanics of Materials, 6/e, F.P. Beer & E. R. Johnston, McGraw Hill - Engineering Mechanics of Solids, 2/e, E. P. Popov, Pearson Education Asia. - Mechanics of Materials, 9/e, R. C. Hibbeler, Prentice Hall.