Bending due to eccentric axial loading in the plane of symmetry It was shown earlier while discussing the stresses in axially loaded members that the axial normal stress produced will be uniform only if the force acts through the centroid of the cross section. Such loads are called centric loads. If it does not, it is called eccentric load. First, let us consider eccentric axial force acting in the plane of symmetry of the cross section.. In such cases, the force should be transferred to the centroid of the cross-section. It can be seen that the eccentric axial load also produces a bending moment as shown. In such cases, we can find out the stresses separately for the axial force and the pure bending moment and using the principle of superposition the net stress can be obtained at any point in the cross-section.. Example 1 Knowing that for the cast iron link shown the allowable stresses are 30 MPa in tension and 120 MPa in compression, determine the maximum force P that can be applied.. Ai mm2; yi mm; Aiyi. 1.; 20×90=1800; 50; 90000. 2.; 40×30=1200; 20; 24000. ΣAi=3000; ΣAiyi=114000. y=ΣAiyiΣAi=38 mm; z. Moment of inertia of the cross-section about the neutral axis (centroidal z-axis) Izz=90×20312+1800×122+30×40312+1200×182 Izz=868×103 mm4=868×10-9 m4; z. The line of action of P is passing through point D in the cross section which is d=28mm below the centroid C. Transferring the force P from D to C by applying two equal and opposite forces of magnitude P at C, we can find the equivalent force-moment system at C. yA yB. The moment Mz=P×d=0.028P clockwise=-0.028P Now, we can see that the effect of P acting eccentrically at D is equivalent to a axial compressive force P at C and pure bending moment Mz=-0.028P The uniform normal stress on the cross section due to the axial compressive force P acting at the centroid, σxxaxial =-PA=-P3000 mm2 =-P3000×10-6 m2 =-333P The normal stress on the cross section due to the bending moment Mz=-0.028P σxxbending=-MzIzzy The stress at B where yB=-0.038 m, σxxbending=--0.028P868×10-9 m4-0.038 m=-1226P The stress at A where yA=+0.022 m, σxxbending=--0.028P868×10-9 m4+0.022 m=+710P Applying the principle of superposition, we can find out the net stress acting on the cross section. σ xxA=-333P+710P=+377P σ xxB=-333P-1226P=-1559P The maximum tensile stress permitted is 30MPa. 30×106=377P≫≫P=79.6×103N The maximum compressive stress permitted is 120MPa. -120×106=-1559P≫≫P=77×103N; y Q. The magnitude of largest force P that can be applied without exceeding either of the allowable stress is the smaller of the two values of P found out. P=77×103N σ xxA=+377P=29 MPa ≪≫ σ xxB=-1559P=120 MPa Note the shift in the neutral axis position. σ xxAy=σ xxB0.06-y ≫≫29y=1200.06-y ≫≫y=0.0117 m=11.7 mm Exercise 1 The vertical portion of the press shown consists of a rectangular tube of wall thickness t=10 mm and outer dimensions 80 mm×60 mm. Knowing that the press has been tightened on wooden planks being glued together until P=20 kN, determine the stress at point A and point B.. Exercise 2 Knowing that the magnitude of horizontal force is P=8 kN, determine the stresses at A and B.. General case of eccentric axial loading In the last section, we discussed about the eccentric axial loading in the plane of symmetry. Now, let us consider the analysis of axial eccentric loading not in the plane of symmetry. Consider a straight beam with a vertical axis of symmetry subjected to an axial force P not in the plane of symmetry.. First, let us transfer the force P to the plane of symmetry and then to the centroid.. The moment My=Pa The moment Mz=Pb. The net stress developed will be the algebraic sum of the stresses produced by the axial force P and the pure bending moments My and Mz (superposition). σxx=±PA±MzIzzy±MyIyyz #Example 1 A vertical 480 kN load is applied as shown on a wooden post of rectangular cross section 80mm x 120 mm. Determine the stress at points A, B, C, and D.. Izz=0.080×0.120312=11.52×10-6 m4 Ixx=0.120×0.08312=5.12×10-6 m4. To begin with, transfer the force P to the plane of symmetry. The force will be accompanied by a couple of moment Mz=480 kN×60mm-35mm=120 N.m. Then, transfer the force to the centroid of the cross section. The accompanying moment is Mx=480 kN×40mm=192 N.m Mz; Mz; 480 kN Mx Mz. The uniform normal stress produced over the cross section ABCD due to the axial force P, σ yyAxial=-PA=-480×103 N0.120 m×0.080 m =-0.5×106 Pa The normal stress on the cross section due to a moment Mz=-120 Nm, σ yybending=-MzIzzx The normal stress at A, D due to the bending moment Mz, where x=-0.06 m σ yybending=--120 N.m11.52×10-6 m4×-0.06 m=-0.625×106 Pa The normal stress at B, C due to the bending moment Mz, where x=+0.06 m σ yybending=--120 N.m11.52×10-6 m4×+0.06 m=+0.625×106 Pa The normal stress on the cross section due to a moment Mx=+192 Nm, σ yybending=-MxIxxz The normal stress at A, B due to the bending moment Mx, where z=+0.04 m σ yybending=-192 N.m5.12×10-6 m4×+0.04 m=-1.5×106 Pa The normal stress at C, D due to the bending moment Mx, where z=-0.04 m σ yybending=-192 N.m5.12×10-6 m4×-0.04 m=+1.5×106 Pa σ yyA=-0.5×106 Pa-0.625×106 Pa-1.5×106 Pa=-2.625×106 Pa σ yyB=-0.5×106 Pa+0.625×106 Pa-1.5×106 Pa=-1.375×106 Pa σ yyC=-0.5×106 Pa+0.625×106 Pa+1.5×106 Pa=+1.625×106 Pa σ yyD=-0.5×106 Pa-0.625×106 Pa+1.5×106 Pa=+0.375×106 Pa. Exercise 1 The tube shown has a thickness 12 mm. Determine the stresses at points A, B, C, and D; C. - Mechanics of Materials, 2/e, J. M. Gere & S. P.Timoshenko, CBS Publishers. - Mechanics of Materials, 6/e, F.P. Beer & E. R. Johnston, McGraw Hill - Engineering Mechanics of Solids, 2/e, E. P. Popov, Pearson Education Asia. - Mechanics of Materials, 9/e, R. C. Hibbeler, Prentice Hall.