Deflection of beams We have already derived the pure bending equation for symmetric, prismatic, homogeneous beams as σxx=-Eρy=-MzIzzy 1ρ=MzEIzz Let v be the deflection of the beam from x-axis downwards. Then v=f(x) is the equation of the elastic curve. From the elementary calculus, the equation for curvature is 1ρ=d2vdx21+dvdx232 For small deformations, the deflection curve is shallow. The slope dvdx=tanθ≅θ(in radians) is small. Then dvdx2 is still smaller. Then 1+dvdx2≅1.; v z y x. 1ρ=d2vdx2=MzEIzz The second order differential equation EIzzd2vdx2=Mz is the equation for the elastic curve. Example 1 Determine the slope and deflection of the cantilever beam loaded at the end.. ΣM1-1=0≪≫M+Px=0≫M=-Px EIzzd2vdx2=Mz=-Px; P V M. Integrating with respect to x, EIzzd2vdx2 dx=-Px dx ≪≫ EIzzdvdx=-Px22+C1 Integrating again EIzzv=-Px36+C1x+C2 Substituting the boundary conditions: At x=L, dv dx=0 and v=0 0=-PL22+C1≫≫ C1=PL22 0=-PL36+PL22L+C2≫≫ C2=-PL33 EIzzdvdx=-Px22+PL22 ≪≫ EIzzv=-Px36+PL22x-PL33 At the free end of the cantilever beam, x=0 dvdx=PL22EIzz ≪≫ v=-PL33EIzz #Example 2 A simply supported beam with uniformly distributed load of intensity w. The total load is wL. The support reactions are wL2. ΣM1-1=0≪≫M-wL2x+wx22=0≫M=wL2x-wx22 EIzzd2vdx2=Mz=wL2x-wx22. Integrating with respect to x, EIzzd2vdx2 dx=wL2x-wx22 dx ≪≫ EIzzdvdx=wLx24-wx36+C1 Integrating again EIzzv=wLx312-wx424+C1x+C2 Substituting the boundary conditions: At x=0, v=0 and at x=L, v=0 0=C2 0=wL412-wL424+C1L≫≫ C1=-wL324 EIzzdvdx=wLx24-wx36-wL324 ≪≫ EIzzv=wLx312-wx424-wL324x When x=L/2 v=-5wL4384EIzz ≪≫ dvdx=0 Exercise 1 Determine the slope and deflection at the free end of a cantilever of length L and carrying a uniformly distributed load of intensity w N/m.. Exercise 2 Determine the deflection of the beam at the centre of a simply supported beam loaded by a concentrated load P at its centre.. Macaulay’s Method Macaulay’s singularity functions can be used to determine the deflection of beams, which can be written as. The integration of the Macaulay’s function follows the same rules as for ordinary functions.. y #Example 1 Determine the equation of the elastic curve for the beam loaded as shown. Also determine the slope and deflection at the free end of the beam.; z x V1 M1 12kN. Section 1-1 ΣM1-1=0≪≫-M1-12x=0≫M1=-12x Section 2-2 ΣM2-2=0≪≫-M2-12x-50-8x-422=0 M2=-12x-50-4x-42; V2 M2 12kN x 8kN/m 4m 50kNm x. Now, we can write a single moment equation for the entire beam using the singularity functions M=-12x-50x-40-4x-42 The deflection equation is EIzzd2vdx2=Mz=-12x-50x-40-4x-42 Integrating with respect to x, EIzzd2vdx2 dx=-12x-50x-40-4x-42 dx EIzzdvdx=-12x22+C1-50x-411-4x-433 Integrating again EIzzv=-6x33+C1x+C2-50x-422-4x-4412 Substituting the boundary conditions: At x=9m, dv dx=0 and v=0 0=-12×922+C1-50(9-4)11-4(9-4)33≫≫ C1=902.7 0=-6×933+902.7×9+C2-509-422-49-4412≫≫ C2=-5832 The equation for the slope of the elastic curve is EIzzdvdx=-6x2+902.7-50x-41-1.33x-43 The equation of the elastic curve is EIzzv=-2x3+902.7x-5832-25x-42-0.33x-44 At the free end of the cantilever (x=0) dvdx=902.7EIzz ≪≫ v=-5832EIzz 8kN #Example 2 Determine the deflections at C and D of the overhanging beam shown.; 120kNm 20m 10m. Let us find out the support reactions first. ΣFx=0≫≫X=0 ΣMA=0≫≫8×10+R×20-120=0 R=2 kN ΣFy=0≫≫Y+R-8=0 ≫≫Y=6 kN; 8kN R Y X 120kNm 20m 10m. Section 1-1 ΣM1-1=0≪≫+M1+8x=0≫M1=-8x Section 2-2 ΣM2-2=0≪≫+M2+8x-6x-10-120x-300=0 M2=-8x+6x-10+120x-300 We know that there is a moment of -120 kNm at x=30 m; V1 M1 8kN x V2 M2 8kN x Y=6 kN. Hence, we can write a single moment equation for the entire beam using the singularity functions as M=-8x+6x-101+120x-300 EIzzd2vdx2=Mz=-8x+6x-101+120x-300 Integrating with respect to x, EIzzdvdx=-8x22+C1+6x-1022+120x-3011 Integrating again EIzzv=-4x33+C1x+C2+3x-1033+120x-3022 Substituting the boundary conditions: At x=10m, v=0 and at x=30m, v=0 0=-4×1033+C1×10+C2≫≫ 10C1+C2=1333.3 0=-4×3033+C1×30+C2+3(30-10)33≫≫ 30C1+C2=28000 Solving, we get C1=1333.3, C2=-12000 The equation for the slope of the elastic curve is EIzzdvdx=-4x2+1333.3+3x-102+120x-30 The equation of the elastic curve is EIzzv=-1.33x3+1333.3x-12000+x-103+60x-302 At the free end of the cantilever (x=0) dvdx=1333.3EIzz ≪≫ vc=-12000EIzz At D, deflection is maximum. Hence, the slope at D (which is at x>10m) should be zero 0=-4x2+1333.3+3x-102≫≫ x2+60x-1633=0≫x=20.3 m The deflection at D vD=+5030EIzz #Example 3 Determine the slope and deflection of the cantilever beam shown.; 8kN/m 5m 4m. The distributed load acting over 4m is assumed to be acting over the entire 9m downwards and acting over 5m upwards as shown (superposition); 8kN/m 8kN/m 5m 4m 8kN/m. Section 1-1 ΣM1-1=0≪≫M1+8x22=0≫M1=-4x2 Section 2-2 ΣM2-2=0≪≫M2+8x22-8x-422=0 M2=-4x2+4x-42; V1 M1 x M21 8kN/m V2 x 4m 8kN/m. we can write a single moment equation for the entire beam using the singularity functions as M=-4x2+4x-42 EIzzd2vdx2=Mz=-4x2+4x-42 Integrating with respect to x, EIzzdvdx=-4x33+C1+4x-433 Integrating again EIzzv=-4x412+C1x+C2+4x-4412 Substituting the boundary conditions: At x=9m,dvdx=0 and v=0 0=-4×933+C1+4(9-4)33≫≫ C1=805.3 0=-4×9412+805.3×9+C2+4(9-4)43≫≫ C2=-5894 The equation for the slope of the elastic curve is EIzzdvdx=-4x33+805.3+4x-433 The equation of the elastic curve is EIzzv=-4x412+805.3x-5894+4x-4412 At the free end of the cantilever (x=0) dvdx=805.3EIzz ≪≫ v=-5894EIzz Exercise 1 Determine the slope and deflection of the cantilever beam at A and B; 8kN/m B A 5m 4m P. Exercise 2 Determine the slope and deflection of the beam directly under the load. What is the maximum deflection and where will it occur?; L b a. Exercise 3 Determine the maximum deflection of the beam loaded as shown.. Exercise 4 Determine the reaction at A of the propped cantilever beam in terms of P, a, L, E, and I. What is the reaction at A and the deflection under the load when a=L2? (Hint: Replace the roller support at A by the reaction RA and find the bending moment equation in terms of Macaulay’s functions and get the expression for slope and deflection as usual. The expressions will have three unknowns C1, C2, and RA. Substitute the three boundary conditions vx=0=0, vx=L=0, and dvdxx=L=0 to evaluate the unknowns.). Exercise 5 Determine the reaction at A of the propped cantilever beam in terms of P, a, L, E, and I. (Hint: Replace the support at A by the reaction RA and moment and MA. Find the bending moment equation in terms of Macaulay’s functions and get the expression for slope and deflection as usual. The expressions will have four unknowns C1, C2, MA, and RA. Substitute the four boundary conditions vx=0=0, vx=L=0, dvdxx=0=0, and dvdxx=L=0 to evaluate the unknowns.). Moment-Area Theorems Consider a beam subjected to some arbitrary loading. The second figure represents the bending moment diagram scaled down by its flexural rigidity i.e., MEI diagram. The third figure shows the elastic deflection curve. We know that the slope, θ=dvdx ∴ d2vdx2=MEI ≫≫ dθdx=MEI≫≫ dθ=MEIdx Considering two arbitrary points C and D on the beam and integrating from C to D θcθDdθ=xcxDMEIdx. θD-θc=xcxDMEIdx The angle between the tangents at D and C θD/C=Area of the MEI diagram between C and D. This is known as the first moment-area theorem. Now consider twwo points P and P’located between C and D.The tangents to the elastic curve at P and P’ intersects the vertical through C making an intercept dt. Since dθ is very small, we can write dt=x1dθ≫≫dt=x1MEIdx. Now we need to integrate this equation from C to D. As P moves from C to D, the tangent at P sweeps a distance CE along the vertical through C denoted by tD/C. tD/C=xcxDMEIdxx1 The vertical offset of point C from the tangent at D is equal to the first moment of the area under the MEI diagram between C and D with respect to the vertical axis through C. tD/C=(Area of MEI diagrm)(x1) This is known as the second moment area theorem.. #Example 1 Determine the slope and deflection at B of the cantilever shown using moment-area method. Take EI as constant.. First draw the bending moment diagram and scale it down to MEI The first moment-area theorem states that θB/C=θB-θA=area ofMEI diagram between A & B θB-θA=12-PLEI×L=-PL22EI. Since, θA=0, θB=-PL22EI. Negative sign shows that the angle is measured clockwise. According to the second moment-area theorem tA/B=(Area of MEI diagram b/w A &B)(x1); tA/B. The vertical offset of point B from the tangent at A is equal to the first moment of the area of MEI diagram between A and B with respect to point B. tA/B=yB=-PL22EI×2L3=-PL33EI #Example 2 Determine the deflection of point D of the simply supported beam shown. Also calculate the slope and deflection of point C.. First sketch the bending moment diagram and scale it down to MEI diagram. The vertical offset of point B from the tangent at D is equal to the first moment of the area of MEI diagram between D & B with respect to point B.. tD/B=+12×PL4EI×L2×23L2=+PL348EI yD=-PL348EI θC/D=θC-θD=area ofMEI diagram between C & D =12×L4×PL4EI+PL8EI=3PL264EI; tD/B. The vertical offset of point C from the tangent at D is equal to the first moment of the area of MEI diagram between D & C with respect to point C. tD/C=PL8EI×L4×L8+12×L4×PL8EI×23L4=+5PL3768EI yC=-PL348EI-5PL3768EI=-11PL3768EI Exercise 1 Determine the slope and deflection of point A of the beam with E=200 GPa and I=65×10-6 m4. Exercise 2 Determine the slope and deflection of the free end of a cantilever beam of length L, flexural rigidity EI carrying a udl of intensity w N/m over the entire length.. Exercise 3 Determine the maximum deflection and slope at the ends of the simply supported beam shown. Exercise 4 Determine the slope and deflection of the beam at the free end.; L L 2EI EI. References - Mechanics of Materials, 2/e, J. M. Gere & S. P.Timoshenko, CBS Publishers. - Mechanics of Materials, 6/e, F.P. Beer & E. R. Johnston, McGraw Hill - Engineering Mechanics of Solids, 2/e, E. P. Popov, Pearson Education Asia. - Mechanics of Materials, 9/e, R. C. Hibbeler, Prentice Hall.