Electron Beam Welding (EBW) is a fusion welding in which coalescence is produced by heating the workpiece due to impingement of the concentrated electron beam of high kinetic energy on the workpiece. As the electron beam impinges the workpiece, kinetic energy of the electron beams converts into thermal energy resulting in melting and even evaporation of the work material.

Principles:

In general, electron beam welding process is carried out in vacuum. In this process, electrons are

emitted from the heated filament called electrode. These electrons are accelerated by applying

high potential difference (30 kV to 175 kV) between cathode and anode. The higher the potential

difference, the higher would be the acceleration of the electrons. The electrons get the speed in

the range of 50,000 to 200,000 km/s. The electron beam is focused by means of electromagnetic

lenses. When this high kinetic energy electron beam strikes on the workpiece, high heat is

generated on the work piece resulting in melting of the work material. Molten metal fills into the

gap between parts to be joined and subsequently it gets solidified and forms the weld joint

EBW Equipment:

An EBW set up consists of the following major equipment:

a) Electron gun,

b) Power supply,

c) Vacuum Chamber,